ДОКУМЕНТ №24

# GIS / Spatial Data / Cadastre / Remote Sensing

Кадастр и LandPlot, производственные Field, версии геометрии, PostGIS, GIS import/export, MapLibre/offline, Sentinel-2, NDVI/NDRE/EVI/NDMI, аномалии, management zones и scouting routes для Codex

Product goal Пользователь должен уметь создать поле вручную, по GIS-файлу или с помощью кадастрового номера/источника; затем видеть актуальную производственную границу, спутниковую динамику и аномалии и одним действием превращать проблему на карте в полевое задание. При этом кадастровая, спутниковая и производственная «истины» не смешиваются.

Версия 1.0 • 170 GIS requirements • 60 acceptance tests • 26 сентября 2026 г.

# 1. Ключевые архитектурные решения

| Decision | Meaning |
| --- | --- |
| LandPlot != Field | Кадастровый участок и производственное поле - разные сущности и могут иметь many-to-many allocation. |
| Geometry is versioned | Новая граница никогда не стирает старую. |
| PostGIS is canonical | Map/tile/mobile caches - производные представления. |
| Cadastre is provider-neutral | НСПД/другой источник подключается через CadastralProvider; неподтверждённый официальный API не выдумываем. |
| Satellite provider-neutral | Copernicus - первый reference provider, но domain не зависит от него. |
| Index != diagnosis | NDVI/NDRE/etc дают сигнал/аномалию; агрономический диагноз требует ground truth/rule. |
| Offline is first-class | Поле/зоны/маршрут и выбранные спутниковые слои доступны без сети. |
| Field-to-task bridge | Anomaly -> AgronomicIssue/ScoutingTask -> GPS/photo/note -> later response comparison. |

# 2. Российский кадастровый контур: текущая позиция

На 26.09.2026 официальный портал НСПД доступен как публичная точка доступа к пространственным данным. В рамках текущей проверки мы не подтверждаем наличие стабильного публичного документированного API, на который можно безопасно завязать production-функцию создания поля. Поэтому архитектура требует capability discovery и поддерживает официальный/лицензионный API, если он подтверждён для внедрения, а также портал/файл/manual fallback.

| Capability | Status in v1 specification | Behavior |
| --- | --- | --- |
| Lookup by cadastral number | REQUIRED interface | Works through provider if API/authorized access available; otherwise assisted/manual/file workflow. |
| Registry geometry fetch | OPTIONAL_PROVIDER_CAPABILITY | Never promised until provider access terms/API verified. |
| Registry attributes | OPTIONAL_PROVIDER_CAPABILITY | Store snapshot/provenance. |
| HTML scraping | NOT_BASELINE | Not allowed as hidden production dependency. |
| File import | REQUIRED FALLBACK | GeoJSON/KML/GPKG/SHP/WKT. |

Важно Даже если геометрия получена из государственного/реестрового источника, она является reference/registry geometry. Активная производственная Field geometry публикуется отдельным governance workflow. Это позволяет честно учитывать фактическую обработку, дороги, лесополосы, исключённые зоны и несколько кадастровых участков.

# 3. Canonical Spatial Data Model

| Entity | Purpose | Key fields |
| --- | --- | --- |
| LandPlot | Registry/cadastral-linked land object | id, tenantId, cadastralNumber?, registrySource?, status |
| LandPlotSnapshot | Immutable provider/import snapshot | landPlotId, source, capturedAt, attributesJson, geometry?, sourceHash |
| Field | Operational agricultural field | id, code, name, activeGeometryVersionId, status |
| FieldGeometryVersion | Versioned production boundary | fieldId, version, geometry, sourceType, sourceRef, status, effectiveAt, area |
| FieldLandPlotAllocation | Relation field<->parcel | fieldId, landPlotId, allocationGeometry?, area?, validFrom/to |
| FieldLineage | Split/merge genealogy | fromFieldId, toFieldId, operation, effectiveAt |
| SpatialImportBatch | GIS file import staging/audit | fileRef, hash, format, sourceCrs, mappingProfile, counts |
| SpatialFeatureStaging | Per-feature preview/validation | importBatchId, sourceFeatureId, geometry, props, issues |
| SpatialQualityIssue | Geometry/data quality issue | entityRef, rule, severity, status, geometry? |
| MapLayerDefinition | Logical map layer | code, sourceType, styleRef, permission, offlinePolicy |
| OfflineMapRegion | Downloaded region metadata | deviceId, scope, bounds/shape, zooms, styleVersion, status |
| RemoteSensingProviderProfile | Provider/API config metadata | provider, apiVersion, capabilities, authRef, licenseRef |
| SatelliteScene | EO scene/item metadata | provider, collection, itemId, capturedAt, footprint, cloudCover, assets |
| RasterAsset | Stored/reference raster asset | sceneId?, objectRef/urlRef, crs, resolution, bands, checksum |
| RasterProcessingJob | Processing state | sceneId, fieldId?, algorithm, version, state, attempts |
| RemoteSensingMetric | Field/zone metric result | sceneId, fieldId, indexCode, formulaVersion, value, coverage |
| RemoteSensingRaster | Derived index raster reference | sceneId, indexCode, formulaVersion, rasterAssetId |
| RemoteSensingAnomaly | Detected anomaly | fieldId, metricRefs, geometry, areaPct, severity, algorithmVersion |
| ManagementZone | Stable/derived zone | fieldId, geometry, zoneType, method, version, status |
| ScoutingRoute | Route from anomalies/issues | taskId, fieldId, routeGeometry, stopRefs, algorithmVersion |
| SoilSamplePoint | Spatial sample | fieldId, point, accuracy, depth, sampledAt, sampleId |
| SpatialAnalysisRun | Generic geoprocessing lineage | operation, inputs, parameters, outputs, version |
| CadastralReconciliation | Official/reference vs operational diff | fieldId, snapshotId, overlap/outside/missing areas, status |
| PrescriptionMap | Variable-rate plan | fieldId, operationId, zones/rates, version, approvalStatus |

# 4. Relation LandPlot <-> Field

LandPlot A -----\
                \
LandPlot B ------> Field 17 ------> FieldSeason 2027 (wheat)
                /
LandPlot C -----/

FieldLandPlotAllocation keeps:
  landPlotId / fieldId
  allocationGeometry?
  allocationAreaHa?
  validFrom / validTo
  source / reason

Поле может включать несколько участков, а производственная граница может исключать часть кадастровой геометрии.

Кадастровая площадь, declared area и area(active operational geometry) показываются отдельно.

Сверка строится геометрически и не исправляет данные сама.

# 5. Создание поля по кадастровому номеру

User enters cadastral number
  -> lexical validation
  -> CadastralProvider.lookup()
       -> API_CONFIRMED / LICENSED_API
       -> or PORTAL/FILE fallback
  -> LandPlotSnapshot
  -> user review
  -> create/link LandPlot
  -> propose Field geometry
  -> compare registry vs operational exclusions/roads/etc
  -> FieldGeometryVersion(PROPOSED)
  -> validate/approve
  -> Field active geometry

| Step | Required behavior |
| --- | --- |
| 1 Search | Never promise remote result if provider capability unavailable. |
| 2 Snapshot | Save retrieval time/source/attributes/geometry/hash. |
| 3 Review | Show what came from provider and what user is creating. |
| 4 Field proposal | One parcel, multi-parcel, or partial allocation. |
| 5 Comparison | Overlap/outside/missing/symmetric-difference area. |
| 6 Publish | Only approved operational geometry becomes active Field boundary. |

# 6. Geometry Versioning / Split / Merge

| Operation | Output | Invariant |
| --- | --- | --- |
| Edit vertices | New FieldGeometryVersion | Old version retained. |
| Adopt cadastral candidate | New geometry version | Snapshot remains immutable source evidence. |
| Split | 2+ Field entities + lineage | No lineage cycles; source historical. |
| Merge | New/survivor Field + lineage | Source history preserved; disjoint may become MultiPolygon. |
| GPS survey | Candidate geometry version | Accuracy/time/device stored. |
| Import | Candidate(s) | Import batch/file/feature provenance retained. |

# 7. CRS / PostGIS policy

| Topic | Decision |
| --- | --- |
| API/GeoJSON | WGS84 longitude/latitude; RFC 7946. |
| Canonical vector | Typed PostGIS geometry in EPSG:4326 for web/domain interchange. |
| Area/distance | PostGIS geography or approved projected work CRS; never Web Mercator. |
| Source CRS | Mandatory metadata for non-4326 sources; transform during staging. |
| Indexes | GiST spatial index for production geometry queries. |
| Validity | ST_IsValid-equivalent; repair creates candidate, does not erase original. |
| Rendering | Simplified/tiles/cache can differ from canonical precision. |

Current PostGIS documentation continues to provide GiST-based spatial indexes and a broad set of spatial processing functions. Exact PostGIS major/minor version is pinned and tested in deployment, not embedded into the domain contract.

# 8. GIS import/export formats

| Format | Direction | Policy |
| --- | --- | --- |
| GeoJSON | Import/export/API | RFC 7946 WGS84 output; strong web interoperability. |
| GeoPackage 1.4 | Import/export/offline package | Preferred rich portable SQLite geospatial exchange; useful for low-connectivity/mobile workflows. |
| KML/KMZ | Import/export | Common farm/GIS exchange; normalize CRS/attributes. |
| ESRI Shapefile | Import/export legacy | Support limitations/encoding/field truncation explicitly. |
| WKT/WKB | Technical/API | Single geometry/technical interchange. |
| GPX | Tracks/waypoints | Scouting/GPS routes where useful. |
| CSV coordinates | Import points/vertices | Explicit column/CRS mapping required. |

OGC GeoPackage 1.4.0 is the current published OGC GeoPackage encoding standard and is specifically designed as a portable SQLite container for vector/raster/tile data, including mobile/offline environments.

# 9. Map Rendering and Offline

MapLibre remains the provider-neutral map renderer. The current flutter-maplibre-gl line in 2026 includes rebuilt offline regions with pause/resume/progress and updated Flutter documentation; the exact plugin version is pinned by release. Offline capability is treated as an implementation contract, not as a reason to tie our data model to MapLibre internals.

| Offline asset | Policy |
| --- | --- |
| BASE_MAP | Tiles/style/assets for configured region and zoom range; provider terms checked. |
| FIELD_VECTOR | Assigned/nearby Field/LandPlot/zone geometry from local DB. |
| WORK_ROUTE | Task/scouting route + stops. |
| REMOTE_SENSING | Selected derived raster/tiles or preview with capturedAt. |
| SATELLITE_PREVIEW | Small cached visualization; date/quality always visible. |

# 10. RemoteSensingProvider contract

RemoteSensingProvider
  capabilities()
  searchScenes(aoi, from, to, filters)
  getSceneMetadata(sceneId)
  resolveAssets(sceneId, bands/qualityLayers)
  requestPreview(...)
  requestStatistics(...) ?
  downloadAsset(...) ?

ProviderProfile
  providerCode
  catalogApiVersion
  processingApiVersion
  authRef
  capabilities[]
  licenseRef
  endpointConfig

# 11. Copernicus / Sentinel-2 baseline

Copernicus Data Space Ecosystem currently exposes STAC v1 at stac.dataspace.copernicus.eu/v1. The legacy catalogue STAC endpoint was deprecated on 17 November 2025. CDSE also provides OData and Sentinel Hub APIs; in March 2026 Sentinel Hub services introduced additional path formats, which is another reason endpoint details stay inside the provider adapter.

Sentinel-2 Level-2A provides atmospherically corrected surface reflectance plus Scene Classification (SCL) and other layers. Copernicus documentation states that Sentinel-2 samples 13 spectral bands at 10/20/60 m, while B04 (Red) and B08 (NIR) are 10 m bands; these are the V1 NDVI baseline.

| Scene field | Required |
| --- | --- |
| provider/collection/itemId | Stable source identity. |
| capturedAt | Acquisition time. |
| footprint | Scene footprint. |
| cloudCover | Provider scene metadata, not field valid coverage. |
| processingLevel/baseline | L2A etc. |
| band assets | Provider asset refs/resolution. |
| quality layers | SCL/dataMask/cloud metadata where available. |
| license/access | Provider profile reference. |

# 12. Vegetation / spectral index registry

| Index | Formula | Baseline/profile | Interpretation boundary |
| --- | --- | --- | --- |
| NDVI | (NIR-RED)/(NIR+RED) | Sentinel baseline B08/B04 at compatible surface reflectance | Greenness/vegetation signal; not diagnosis. |
| GNDVI | (NIR-GREEN)/(NIR+GREEN) | Typical Sentinel B08/B03 | Vegetation/chlorophyll-related signal; profile-dependent interpretation. |
| NDRE | (NIR-RED_EDGE)/(NIR+RED_EDGE) | Band mapping profile required | Later-stage/red-edge-sensitive vegetation signal. |
| EVI | 2.5*(NIR-RED)/(NIR+6*RED-7.5*BLUE+1) | Typical B08/B04/B02 | Vegetation signal with blue-band correction. |
| SAVI | (1+L)*(NIR-RED)/(NIR+RED+L) | L profile required | Soil-adjusted vegetation index. |
| MSAVI2 | Formula Registry KPI-RS-006 | NIR/RED | Soil influence reduction. |
| NDMI | (NIR-SWIR1)/(NIR+SWIR1) | Band mapping/resampling explicit | Canopy/moisture-related signal. |
| NDWI | (GREEN-NIR)/(GREEN+NIR) | Semantic variant named | Water-related index variant. |
| NBR | (NIR-SWIR2)/(NIR+SWIR2) | Band mapping profile | Burn/disturbance indicator; optional agriculture context. |

No “magic NDVI” Индекс - это измерительный/аналитический слой. Низкое значение может быть связано с облаками, фазой развития, почвой, влагой, пропусками, болезнью, питанием и множеством других причин. Система должна направлять агронома к проверке, а не выдавать необоснованный диагноз.

# 13. Cloud / quality masking

| Stage | Rule |
| --- | --- |
| Scene filter | Provider scene-level cloudCover is coarse prefilter only. |
| Pixel mask | Use quality layer/SCL/cloud-shadow/no-data/saturation mask profile. |
| Field crop | Clip/mask to Field geometry version effective for analysis. |
| Valid coverage | validPixelPct calculated for field/zone. |
| Threshold | LOW_COVERAGE if below Formula/RemoteSensingProfile threshold. |
| Storage | Metric retains maskProfileVersion and scene source. |

Copernicus documents SCL-based masking for its Sentinel-2 quarterly mosaic, including saturated/defective, cloud shadow, cloud probability and cirrus classes. Our mask profile is versioned because the exact acceptable classes can differ by product/use case.

# 14. Raster processing architecture

STAC/Catalog search
      -> SatelliteScene metadata
      -> resolve remote assets
      -> raster worker (Python)
           window/intersection read
           surface reflectance / quality mask
           resample if required (record method)
           calculate index
           zonal statistics
      -> MinIO/S3 derived raster (optional)
      -> PostgreSQL MetricResult / anomaly metadata
      -> Map tiles/preview/cache

NestJS orchestrates jobs/status; Python raster worker handles raster/numeric processing.

COG-like tiled/range-readable assets are preferred when persisted internally.

No full-scene raster in HTTP process memory.

# 15. Time Series / Anomaly / Management Zones

| Layer | Result | Required provenance |
| --- | --- | --- |
| Scene metric | Mean/median/percentiles/CV | scene + formula + mask + coverage. |
| Time series | Field index history | compatible formula/profile basis. |
| Temporal delta | Current - previous/reference | dates + compatibility. |
| Peer anomaly | z-score/relative deviation | peer-group definition/version. |
| Spatial anomaly | Polygon/mask + area share | source raster/algorithm version. |
| Management zone | Stable polygon classes | multi-layer inputs/method/version/status. |

# 16. Satellite anomaly -> field work

New compatible scene
  -> calculate metrics
  -> anomaly detector
  -> RemoteSensingAnomaly
  -> rule/review
  -> NotificationCase
  -> [Create scouting task]
  -> mobile offline route
  -> GPS stop / photo / voice / note / sample
  -> AgronomicIssue / recommendation / treatment
  -> later scene/observation comparison

Competitive differentiator Мы связываем спутниковую аналитику не с красивой картинкой, а с операционным workflow: обнаружить -> проверить в поле -> зафиксировать доказательство -> назначить действие -> оценить результат. Это должно быть измеримо по времени от anomaly detection до ground-truth observation.

# 17. Agrochemical / soil / yield / application maps

| Map type | Canonical source | Derived layer |
| --- | --- | --- |
| Soil samples | Sample + Lab Result + GPS accuracy | Point/classes/interpolated surface. |
| Yield | Raw yield/GPS/device data | Cleaned yield map/zones. |
| Planned prescription | Agronomic approved PrescriptionMap | Vendor-machine export. |
| Actual application | Machine/manual OperationFact + track/rate | Coverage/rate/overlap/skip. |
| Weather | Station/provider/model | Spatial/context overlay only. |
| Issues/scouting | Observation/AgronomicIssue | Heatmap/points/zones. |

# 18. Workflow Catalog

| ID | Workflow | Definition |
| --- | --- | --- |
| WF-GIS-01 | Create field manually | Draw/import polygon -> validate -> draft -> approve -> Field + GeometryVersion. |
| WF-GIS-02 | Create from cadastral number | Enter cadastral number -> CadastralProvider lookup/fallback -> snapshot -> review -> LandPlot -> propose Field -> approve. |
| WF-GIS-03 | Create field from several parcels | Select LandPlots -> choose full/partial allocation -> union/proposed boundary -> review discrepancies -> approve. |
| WF-GIS-04 | Update cadastral snapshot | Refresh provider -> new snapshot -> diff -> reconciliation -> optionally adopt candidate geometry. |
| WF-GIS-05 | Split field | Select active geometry -> draw split line/polygons -> validate -> impact preview -> create new Fields/lineage -> archive source. |
| WF-GIS-06 | Merge fields | Select fields -> validate tenant/season/business constraints -> union/MultiPolygon -> lineage -> publish new/survivor field. |
| WF-GIS-07 | GPS boundary | Capture fixes+accuracy -> clean candidate -> compare existing -> review -> publish geometry version. |
| WF-GIS-08 | GIS bulk import | Upload -> detect format/layers/CRS -> map fields -> preview -> validate -> dry run -> commit valid/all. |
| WF-RS-01 | Satellite scene discovery | Field geometry/date -> provider STAC/search -> candidate scenes -> quality filter -> register scene. |
| WF-RS-02 | Compute NDVI | Scene L2A assets -> mask -> band mapping -> index raster -> zonal stats -> MetricResult. |
| WF-RS-03 | Detect anomaly | Time-series/peer/profile -> algorithm -> anomaly geometry/severity -> notification/task proposal. |
| WF-RS-04 | Scout anomaly | Open anomaly offline -> route -> navigate -> GPS/photo/note -> issue/task -> sync. |
| WF-RS-05 | Assess response | Treatment/task -> later compatible scenes/observations -> compare metrics -> evidence report. |
| WF-AGMAP-01 | Prescription map | Input layers -> agronomic rule/model -> zones/rates -> agronomist review -> approve -> vendor export. |

# 19. GIS / Remote Sensing API

| Method | Path | operationId | Purpose |
| --- | --- | --- | --- |
| GET | /v1/fields/{id}/geometry | fieldGeometryGet | Active/versioned geometry with provenance. |
| POST | /v1/fields | fieldCreate | Create operational Field shell/draft. |
| POST | /v1/fields/{id}/geometry-versions | fieldGeometryVersionCreate | Submit candidate geometry. |
| POST | /v1/field-geometry-versions/{id}/commands/approve | fieldGeometryApprove | Publish active geometry. |
| POST | /v1/fields/{id}/commands/split | fieldSplit | Split workflow. |
| POST | /v1/fields/commands/merge | fieldMerge | Merge workflow. |
| POST | /v1/cadastre/lookups | cadastreLookup | Provider-neutral cadastral lookup. |
| POST | /v1/land-plots | landPlotCreate | Create/link LandPlot from reviewed snapshot/import. |
| POST | /v1/spatial-imports | spatialImportCreate | Start GIS import staging. |
| POST | /v1/spatial-imports/{id}/commands/commit | spatialImportCommit | Commit selected valid features. |
| GET | /v1/map/layers | mapLayerList | Permission-aware layer definitions. |
| POST | /v1/remote-sensing/scenes/search | remoteSceneSearch | Provider search. |
| POST | /v1/remote-sensing/jobs | remoteProcessingJobCreate | Index/statistics processing. |
| GET | /v1/fields/{id}/remote-sensing/series | remoteSeriesGet | Field time series. |
| GET | /v1/fields/{id}/anomalies | remoteAnomalyList | Anomalies. |
| POST | /v1/anomalies/{id}/commands/create-scouting-task | anomalyCreateScoutingTask | Field workflow bridge. |
| POST | /v1/prescription-maps | prescriptionMapCreate | Create draft prescription. |
| POST | /v1/prescription-maps/{id}/commands/publish | prescriptionMapPublish | Approve/version prescription. |

# 20. Machine-readable examples

field_geometry_version:
  id: uuid
  fieldId: uuid
  version: 7
  geometry:
    type: MultiPolygon
    srid: 4326
  source:
    type: CADASTRE_REFERENCE
    refId: land_plot_snapshot_uuid
  status: PROPOSED
  measurement:
    areaMethod: POSTGIS_GEOGRAPHY
    geometricAreaHa: 128.4421
  qualityFlags: []

remote_sensing_profile:
  provider: COPERNICUS_CDSE
  collection: sentinel-2-l2a
  catalog:
    type: STAC
    endpointProfileVersion: 2026-09
  indexProfiles:
    NDVI:
      nir: B08
      red: B04
      targetResolutionM: 10
  maskProfile: S2_L2A_SCL_V1
  minValidPixelPct: 70

anomaly_policy:
  code: NDVI_TEMPORAL_DROP
  formula: KPI-RS-019
  compareWindowDays: 14
  minValidPixelPct: 70
  thresholdProfile: crop_stage_specific
  action:
    createCase: true
    suggestScoutingTask: true
    autoDiagnosis: false

# 21. Full GIS Requirements Registry

| Area | Count |
| --- | --- |
| AGRO_MAPS | 10 |
| ANALYTICS | 15 |
| CADASTRE | 20 |
| CRS_POSTGIS | 20 |
| DOMAIN | 20 |
| GEOMETRY_EDIT | 15 |
| IMPORT_EXPORT | 15 |
| INDICES | 15 |
| MAP_OFFLINE | 15 |
| OPERATIONS | 10 |
| REMOTE_SENSING | 15 |
| TOTAL | 170 |

## 21.1. AGRO_MAPS

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-151 | Soil sample points | Sample point has GPS accuracy/depth/date/lab linkage. | MUST |
| GIS-152 | Soil analysis layer | Lab results can render point/class/interpolated layer; interpolation method/version explicit. | MUST |
| GIS-153 | Interpolation not raw fact | Kriging/IDW/other surface is derived layer; original sample results preserved. | MUST |
| GIS-154 | Yield map | Combine harvester/yield data requires position/time/quality cleaning profile before map. | MUST |
| GIS-155 | Application map | Actual machine coverage/rate map separate from planned prescription. | MUST |
| GIS-156 | Prescription map | Zone/rate prescription versioned, approved and linked to TechCard/Operation. | MUST |
| GIS-157 | Prescription export adapter | Machine format export is vendor adapter; canonical prescription geometry/rate remains platform model. | MUST |
| GIS-158 | Overlap/skip geometry | Operation coverage analytics compute overlap/uncovered area against target Field/zone. | MUST |
| GIS-159 | Weather overlay | Weather station/model values may be linked/displayed spatially but source/model provenance retained. | MUST |
| GIS-160 | Layer stacking | User can compare satellite/soil/yield/application/issues without merging them into one uncontrolled dataset. | MUST |

## 21.2. ANALYTICS

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-136 | Field zonal stats | For each valid scene/index calculate mean/median/stddev/CV/percentiles/valid coverage as configured. | MUST |
| GIS-137 | Time series | Store one MetricResult per field/scene/index/profile; derived seasonal series queryable. | MUST |
| GIS-138 | Scene age visible | UI/report always shows capturedAt and processingAt. | MUST |
| GIS-139 | Temporal delta | Compare only compatible formula/band/profile/resolution basis or mark mixed basis. | MUST |
| GIS-140 | Peer comparison | Peer group criteria crop/region/stage/season explicit; z-score is analytic, not agronomic truth. | MUST |
| GIS-141 | Anomaly detector versioned | Threshold/statistical/ML anomaly method stores algorithmVersion and inputs. | MUST |
| GIS-142 | Anomaly geometry | Persist polygon/mask reference + areaShare + severity + sourceMetricRefs. | MUST |
| GIS-143 | Management zones | Zones can derive from multi-date/multi-layer clustering; method/version retained. | MUST |
| GIS-144 | Zone stability | Single noisy scene cannot silently replace stable management zone without configured policy. | MUST |
| GIS-145 | Anomaly workflow | Detected anomaly -> review/rule -> AgronomicIssue/ScoutingTask proposal. | MUST |
| GIS-146 | Scouting route | Route builder orders anomaly points/zones and respects field access/start point; route is task aid, not navigation safety guarantee. | MUST |
| GIS-147 | Ground truth linkage | Observation/photo/GPS from scouting links back to anomaly and scene. | MUST |
| GIS-148 | Measure response | Treatment/task outcome can be compared with later index/field observation, preserving causal uncertainty. | MUST |
| GIS-149 | No automatic prescription by default | Variable-rate/prescription map requires explicit agronomic rule/profile/approval. | MUST |
| GIS-150 | Alert dedupe | Repeated overlapping anomalies within cooldown/group policy create/update one case rather than notification flood. | MUST |

## 21.3. CADASTRE

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-041 | Provider-neutral cadastral port | Implement CadastralProvider, not direct dependency from Field domain to one portal. | MUST |
| GIS-042 | Search by cadastral number | Provider contract supports lookup(cadastralNumber) where legally/technically available. | MUST |
| GIS-043 | Current Russian baseline | NSPD public portal is a recognized public spatial-data access point, but no stable public official API is assumed without verified documentation/contract. | MUST |
| GIS-044 | API availability flag | Provider capability records API_CONFIRMED, LICENSED_API, PORTAL_ONLY, FILE_IMPORT, DISCOVERY_REQUIRED. | MUST |
| GIS-045 | No scraping as core dependency | Do not build production cadastral integration on brittle HTML scraping unless separately approved/legal and isolated. | MUST |
| GIS-046 | Portal/file fallback | If direct API unavailable, support manual lookup + GeoJSON/KML/GPKG/SHP/WKT import with provenance. | MUST |
| GIS-047 | Lookup snapshot | Store provider name, cadastral number, retrieval time, raw/ref hash, available attributes and geometry. | MUST |
| GIS-048 | No legal-title inference | Portal/registry spatial data does not by itself establish ownership/rights inside platform. | MUST |
| GIS-049 | Create LandPlot from lookup | User reviews snapshot then creates/links LandPlot; no silent auto-creation into active production scope. | MUST |
| GIS-050 | Create Field from LandPlot | Wizard can propose Field geometry/name/area from one or multiple LandPlots. | MUST |
| GIS-051 | Multi-parcel Field | Union/proposed operational boundary can use several cadastral parcels; relation retains all sources. | MUST |
| GIS-052 | Partial parcel allocation | Field may use part of LandPlot through allocation geometry/area; must not pretend whole parcel is cultivated. | MUST |
| GIS-053 | Official vs actual boundary comparison | Calculate intersection, outside area, missing area, symmetric-difference area and percentage. | MUST |
| GIS-054 | Cadastral number validation | Validate lexical format separately from remote existence lookup. | MUST |
| GIS-055 | Provider outage | Previously saved cadastral snapshots remain usable and visibly dated. | MUST |
| GIS-056 | Provider data licensing | Source terms/license/access level stored in CadastralProviderProfile where required. | MUST |
| GIS-057 | Bulk cadastral import | Support batch of cadastral numbers; unresolved items become issues, not fatal all-or-nothing unless requested. | MUST |
| GIS-058 | Registry refresh | New lookup creates new snapshot and diff; does not mutate old snapshot. | MUST |
| GIS-059 | Cadastral reconciliation issue | Difference has OPEN/REVIEWED/ACCEPT_OPERATIONAL/ADOPT_PROPOSAL/IGNORE_WITH_REASON status. | MUST |
| GIS-060 | Advanced feature tag | Cadastral-assisted field creation is tagged ADVANCED_FEATURE and COMPETITIVE_DIFFERENTIATOR. | MUST |

## 21.4. CRS_POSTGIS

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-021 | Canonical API geometry | GeoJSON/API exchange uses WGS84 longitude/latitude consistent with RFC 7946. | MUST |
| GIS-022 | Database canonical vector geometry | Store canonical web-facing geometry in geometry(MultiPolygon,4326) or equivalent typed geometry. | MUST |
| GIS-023 | Measurement is not EPSG:3857 | Area/distance never calculated from Web Mercator. | MUST |
| GIS-024 | Area measurement method | Use PostGIS geography for broad correctness or transform to approved local/projected CRS profile; record method/version. | MUST |
| GIS-025 | Source CRS required | Non-4326 import must provide/detect CRS; unknown CRS cannot be silently assumed. | MUST |
| GIS-026 | Geometry validity | Import/edit passes ST_IsValid-equivalent validation; invalid geometry becomes issue/proposal. | MUST |
| GIS-027 | MakeValid is controlled | Automatic repair may create candidate geometry but must preserve original/source and repair log. | MUST |
| GIS-028 | Polygon normalization | Operational Field stored Polygon/MultiPolygon; unsupported GeometryCollection requires conversion/review. | MUST |
| GIS-029 | Ring orientation not business identity | Normalize on serialization if needed; equality uses spatial semantics/profile, not raw coordinate order. | MUST |
| GIS-030 | GiST spatial indexes | Production bbox/intersects/within queries use spatial indexes and are performance-tested. | MUST |
| GIS-031 | Bbox prefilter | Expensive spatial operations use indexed bbox/filter where PostGIS planner supports. | MUST |
| GIS-032 | Simplified render geometry | Zoom-specific generalized geometry/read model may be generated; canonical geometry remains untouched. | MUST |
| GIS-033 | Topology-preserving simplification | Use topology-preserving simplification for polygon rendering where appropriate. | MUST |
| GIS-034 | Precision policy | Coordinate precision is not arbitrarily truncated in canonical storage; export profile may round. | MUST |
| GIS-035 | Antimeridian handling | GeoJSON/world provider functions must not assume every polygon avoids antimeridian; Russian production use still validated generically. | MUST |
| GIS-036 | Z/M handling | 2D operational polygons baseline; Z/M preserved/imported only where explicitly supported. | MUST |
| GIS-037 | Spatial query limits | API caps bbox area/feature count/vertex count to prevent expensive unrestricted queries. | MUST |
| GIS-038 | Geometry hash | Version may store normalized geometry hash for dedup/change detection. | MUST |
| GIS-039 | Area recalculation | Geometry approval recalculates geometric area and dependent spatial metrics. | MUST |
| GIS-040 | PostGIS is spatial authority | No duplicated independent geometry truth in Elasticsearch/tiles/mobile cache. | MUST |

## 21.5. DOMAIN

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-001 | LandPlot and Field are distinct | LandPlot represents cadastral/legal/registry-linked land; Field represents operational agricultural production boundary. | MUST |
| GIS-002 | Field may span multiple LandPlots | Use FieldLandPlotAllocation relation with effective dates and optional allocated geometry/area. | MUST |
| GIS-003 | LandPlot may contribute to multiple operational fields | Allowed only through explicit allocation/version rules; overlap must be explainable and time-scoped. | MUST |
| GIS-004 | FieldSeason is separate | Crop/variety/year never overwrite long-lived Field identity. | MUST |
| GIS-005 | Geometry is versioned | Field geometry changes create FieldGeometryVersion; no in-place destructive replacement. | MUST |
| GIS-006 | Active geometry pointer | Field references exactly one approved active geometry version per effective instant. | MUST |
| GIS-007 | Declared and geometric area are separate | declared_area_ha is business/reference attribute; geometric_area_ha is derived from geometry. | MUST |
| GIS-008 | Canonical field ID survives boundary edits | Geometry edit does not create a new Field unless business identity is intentionally split/merged. | MUST |
| GIS-009 | Field split creates lineage | SplitFieldCommand creates new Fields plus FieldLineage edges; source field archived/superseded per workflow. | MUST |
| GIS-010 | Field merge creates lineage | MergeFieldsCommand creates survivor/new field plus lineage; source fields remain historical. | MUST |
| GIS-011 | No cyclic field lineage | Split/merge genealogy is a DAG. | MUST |
| GIS-012 | Geometry source is explicit | MANUAL, IMPORT, CADASTRE_REFERENCE, GPS_SURVEY, DEVICE_TRACK, EXTERNAL_API, DERIVED. | MUST |
| GIS-013 | Geometry status is explicit | PROPOSED, VALIDATING, REVIEW, APPROVED, REJECTED, SUPERSEDED. | MUST |
| GIS-014 | Geometry approval can be role-gated | Tenant policy may require GIS/agronomy approval before active boundary changes. | MUST |
| GIS-015 | Historical facts retain geometry version | Operation/observation/report may store geometryVersionId when spatial interpretation depends on boundary at the time. | MUST |
| GIS-016 | External cadastral identity is not platform PK | Cadastral number is unique reference within source/profile/effective context; platform UUID remains identity. | MUST |
| GIS-017 | Cadastral snapshot immutable | Retrieved registry metadata/geometry stored as source snapshot with capturedAt/provider/version. | MUST |
| GIS-018 | No registry overwrite of operational field | New cadastral snapshot creates reconciliation proposal, never silent replacement. | MUST |
| GIS-019 | Spatial quality metadata required | source, CRS, accuracy, capturedAt, method and quality flags preserved where available. | MUST |
| GIS-020 | Tenant isolation applies to all spatial assets | No cross-tenant spatial query or tile cache leakage. | MUST |

## 21.6. GEOMETRY_EDIT

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-076 | Draw field | Mobile/web can create polygon by map drawing with snap/undo/validation. | MUST |
| GIS-077 | GPS walk/drive boundary | Capture point accuracy and time; create candidate geometry, not automatic approved boundary. | MUST |
| GIS-078 | Edit vertices | Every edit creates draft/version; approval publishes. | MUST |
| GIS-079 | Hole support | Fields may contain excluded interior areas; holes preserved. | MUST |
| GIS-080 | Self-intersection rejection | Invalid self-intersecting polygon cannot become active without valid repair. | MUST |
| GIS-081 | Tiny slivers | Sliver/short segment detection configurable; warn rather than silently delete unless repair command chosen. | MUST |
| GIS-082 | Split by line | Split operation validates resulting polygons/min area and creates lineage. | MUST |
| GIS-083 | Merge adjacent/non-adjacent | Merge allows union only under configured geometry policy; disjoint may produce MultiPolygon. | MUST |
| GIS-084 | Buffer operation | Buffer is analytical/tooling action; cannot silently redefine Field without version approval. | MUST |
| GIS-085 | Difference/intersection | Available for proposals/zones/reconciliation; outputs store operation provenance. | MUST |
| GIS-086 | Snap tolerance | Snap uses explicit meter/CRS tolerance, never screen-pixel semantics for canonical edit. | MUST |
| GIS-087 | Undo/redo | Client edit history does not bypass server validation. | MUST |
| GIS-088 | Concurrent edit | Optimistic version check; geometry conflict never last-write-wins. | MUST |
| GIS-089 | Area delta warning | Publish displays old/new area and percent delta above threshold. | MUST |
| GIS-090 | Affected references | Split/merge wizard reports impacted FieldSeason/tasks/zones/history before commit. | MUST |

## 21.7. IMPORT_EXPORT

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-061 | GeoJSON import/export | Support RFC 7946 GeoJSON; export WGS84 lon/lat. | MUST |
| GIS-062 | KML/KMZ | Support common field/track import/export; normalize to canonical geometry. | MUST |
| GIS-063 | GeoPackage | Preferred rich portable offline/interchange package; support vector layers and metadata. | MUST |
| GIS-064 | Shapefile | Support legacy SHP bundle import/export with encoding/field-name limitations surfaced. | MUST |
| GIS-065 | WKT/WKB | Support technical admin/API import/export for single geometry. | MUST |
| GIS-066 | CSV coordinates | Support points/vertices only with explicit column mapping and CRS. | MUST |
| GIS-067 | GPX | Support tracks/waypoints where agronomy/fleet workflows need it. | MUST |
| GIS-068 | Import staging | File -> inspect layers/CRS -> preview -> map attributes -> validate -> commit. | MUST |
| GIS-069 | File hash | Import batch stores SHA-256 and original file metadata. | MUST |
| GIS-070 | Row/feature provenance | Committed geometry links to import batch and source feature identifier/index. | MUST |
| GIS-071 | Dry run | Bulk import can validate without committing. | MUST |
| GIS-072 | Partial success policy | Per-feature errors available; commit mode ALL_OR_NOTHING or VALID_ONLY explicitly selected. | MUST |
| GIS-073 | Duplicate detection | Spatial/natural-key similarity creates candidates; no automatic destructive merge. | MUST |
| GIS-074 | Export metadata | GIS export includes IDs, geometryVersion, source/effective date and optionally attributes permitted by role. | MUST |
| GIS-075 | Large export async | Large GPKG/SHP/GeoJSON package generated as async job. | MUST |

## 21.8. INDICES

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-121 | NDVI | FormulaRegistry KPI-RS-001; provider band profile maps NIR/RED. | MUST |
| GIS-122 | Sentinel-2 NDVI baseline | For 10m profile use B08 NIR and B04 Red surface reflectance. | MUST |
| GIS-123 | GNDVI | NIR and Green mapping versioned; Sentinel profile typically B08/B03. | MUST |
| GIS-124 | NDRE | Red-edge choice is FormulaProfile/provider-specific; do not hard-code one band pair globally. | MUST |
| GIS-125 | EVI | Blue/Red/NIR formula from FormulaRegistry; provider band mapping explicit. | MUST |
| GIS-126 | SAVI/MSAVI2 | Soil-adjustment coefficient/profile stored. | MUST |
| GIS-127 | NDMI | NIR/SWIR band mapping and resampling explicit. | MUST |
| GIS-128 | NDWI | Index semantic variant named explicitly (e.g., Green-NIR water index) to avoid ambiguity. | MUST |
| GIS-129 | NBR | Available for selected monitoring use, not automatically interpreted agronomically. | MUST |
| GIS-130 | Reflectance input | Indices calculated on appropriate reflectance values, not raw display RGB. | MUST |
| GIS-131 | Cloud/shadow mask | Mask invalid cloud/shadow/saturated/no-data pixels before zonal statistics. | MUST |
| GIS-132 | SCL support | Sentinel-2 L2A Scene Classification Layer may drive mask profile; mask classes versioned/configurable. | MUST |
| GIS-133 | Valid pixel threshold | Metric below profile validPixelPct threshold receives LOW_COVERAGE and may not trigger critical alert. | MUST |
| GIS-134 | No disease diagnosis from index alone | NDVI/other index anomaly is evidence for inspection, not disease/fertilizer prescription by itself. | MUST |
| GIS-135 | Index range validation | Expected theoretical ranges checked; impossible values flag processing issue. | MUST |

## 21.9. MAP_OFFLINE

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-091 | MapLibre renderer | Flutter/web map uses MapLibre-compatible styles/sources; provider-neutral. | MUST |
| GIS-092 | Flutter plugin profile | Pin supported flutter-maplibre-gl/MapLibre integration version per release; no unbounded dependency drift. | MUST |
| GIS-093 | Offline regions | Android/iOS support downloadable offline regions with pause/resume/progress using supported MapLibre offline capability. | MUST |
| GIS-094 | Offline pack metadata | Pack stores tenant, scope, bbox/shape, zooms, styleVersion, createdAt, lastValidatedAt, bytes. | MUST |
| GIS-095 | Field vectors offline | Assigned/nearby fields and relevant zones cached in local DB independent from basemap tiles. | MUST |
| GIS-096 | Satellite preview offline | Selected processed layer/preview may be cached with capturedAt; never presented as live/current without date. | MUST |
| GIS-097 | No provider license violation | Tile/offline caching respects provider terms; self-host/open/licensed sources preferred for production offline. | MUST |
| GIS-098 | PMTiles optional | Portable tile file may be used for offline/base layers behind MapSource adapter. | MUST |
| GIS-099 | Vector style versioning | Style JSON/source version is part of offline pack compatibility metadata. | MUST |
| GIS-100 | Large GeoJSON avoidance | High feature counts use vector tiles/simplified sources/clustering rather than one giant GeoJSON payload. | MUST |
| GIS-101 | Feature state | Use feature-state/restyle path where possible rather than re-sending whole geometry collection. | MUST |
| GIS-102 | Offscreen map pause | Mobile map rendering paused when offscreen where supported to save battery/GPU. | MUST |
| GIS-103 | Map scale/coordinates | Display scale/coordinates where role needs; coordinate format configurable. | MUST |
| GIS-104 | Layer permissions | Unauthorized layer/features excluded server-side, not merely hidden in style. | MUST |
| GIS-105 | Map attribution | Source attribution/licensing displayed according to provider requirements. | MUST |

## 21.10. OPERATIONS

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-161 | Spatial permissions | Field/layer/tile queries enforce role/scope server-side. | MUST |
| GIS-162 | Location privacy | Employee-linked GPS obeys personal-data purpose/access/retention profile. | MUST |
| GIS-163 | Signed URLs | Raster/file assets use authorized short-lived access; public buckets forbidden. | MUST |
| GIS-164 | Provider credentials | Satellite/cadastral/tile credentials in SecretStore, never mobile bundle unless public token intentionally scoped. | MUST |
| GIS-165 | Import parser isolation | GIS files size/feature/decompression/path traversal limits enforced. | MUST |
| GIS-166 | Performance profile | 10k fields bbox query P95 <=1.5s ENTERPRISE per Doc20. | MUST |
| GIS-167 | Large geometry | Typical <=5k vertices synchronous; oversized geometry follows bounded validation/simplification path. | MUST |
| GIS-168 | Raster queues | Provider/raster backlog observable with scene/field/job status. | MUST |
| GIS-169 | Cache invalidation | Geometry/style/layer version change invalidates affected tile/offline cache version. | MUST |
| GIS-170 | Audit | Approve/reject/split/merge/adopt-cadastral/prescription publish actions audited. | MUST |

## 21.11. REMOTE_SENSING

| ID | Requirement | Decision | Level |
| --- | --- | --- | --- |
| GIS-106 | Provider-neutral interface | RemoteSensingProvider search/download/process/statistics methods isolate Copernicus/other providers. | MUST |
| GIS-107 | Current Copernicus catalog baseline | CDSE STAC v1 endpoint is current baseline; legacy catalogue STAC endpoint was deprecated 17 Nov 2025. | MUST |
| GIS-108 | Sentinel-2 L2A preferred baseline | Use atmospherically corrected surface reflectance for vegetation analytics where available. | MUST |
| GIS-109 | Scene metadata | Store provider, collection, itemId, capturedAt, footprint, cloudCover, processingLevel/baseline, assets, license. | MUST |
| GIS-110 | Scene quality | Store field-level validPixelPct separately from provider whole-scene cloudCover. | MUST |
| GIS-111 | Scene discovery by geometry/time | STAC search uses intersects/bbox + datetime and cloud-cover filters as provider supports. | MUST |
| GIS-112 | Provider API versioning | Endpoint/catalog/processing API version pinned in provider profile. | MUST |
| GIS-113 | Provider endpoint changes isolated | 2026 Sentinel Hub path changes cannot leak into domain API. | MUST |
| GIS-114 | Raster originals optional retention | Source product may be referenced/downloaded/cached based on licensing/storage policy. | MUST |
| GIS-115 | COG preferred internal raster form | Where we persist derived rasters, prefer cloud-optimized/tiled GeoTIFF-compatible layout or provider-neutral object asset. | MUST |
| GIS-116 | Raster processing workers | Heavy processing runs in isolated worker/FastAPI/Python path, not NestJS HTTP request. | MUST |
| GIS-117 | Windowed processing | Read only intersecting windows/tiles for field operations; avoid loading full scenes. | MUST |
| GIS-118 | Pixel grid provenance | Resolution/resampling method/CRS are stored with derived raster. | MUST |
| GIS-119 | No resampling deception | Index from mixed-resolution bands records resampling method/target resolution. | MUST |
| GIS-120 | Advanced feature tag | Satellite time-series/anomaly/scouting integration marked ADVANCED_FEATURE and COMPETITIVE_DIFFERENTIATOR. | MUST |

# 22. Acceptance Test Suite

| Test | Area | Expected |
| --- | --- | --- |
| GIS-T001 | LandPlot != Field | Creating cadastral parcel does not automatically create/overwrite operational Field. |
| GIS-T002 | One field many parcels | Field links to 3 LandPlots and retains allocations. |
| GIS-T003 | Partial allocation | Only allocated subgeometry counts toward Field<->LandPlot coverage. |
| GIS-T004 | FieldSeason separation | Crop change does not mutate Field master identity. |
| GIS-T005 | Geometry version | Approved edit creates new version; old geometry remains queryable. |
| GIS-T006 | Concurrent edit | Stale baseVersion geometry publish returns conflict, no LWW. |
| GIS-T007 | Self-intersection | Invalid polygon cannot become active. |
| GIS-T008 | Repair provenance | MakeValid candidate preserves original and repair metadata. |
| GIS-T009 | Area method | EPSG:3857 is never used as authoritative area calculation. |
| GIS-T010 | CRS missing | Non-GeoJSON import with unknown CRS requires user mapping/reject. |
| GIS-T011 | GeoJSON export | Coordinates export RFC7946 lon/lat WGS84. |
| GIS-T012 | GiST use | Representative bbox/intersection query uses spatial index and meets NFR. |
| GIS-T013 | Simplification | Render simplification does not mutate canonical geometry/hash. |
| GIS-T014 | Cadastral lookup capability | Provider marked DISCOVERY_REQUIRED/PORTAL_ONLY cannot be invoked as fictional API. |
| GIS-T015 | Cadastral snapshot | Refresh creates second immutable snapshot and diff. |
| GIS-T016 | No cadastral silent overwrite | New reference geometry creates reconciliation issue only. |
| GIS-T017 | Create field by cad number | Lookup/import -> LandPlot -> reviewed Field proposal -> approved Field. |
| GIS-T018 | Bulk cadastral numbers | One unresolved number does not erase successfully staged items. |
| GIS-T019 | Official vs operational diff | Intersection/outside/missing/symmetric-difference values reconcile geometrically. |
| GIS-T020 | Split | Split creates valid child fields and FieldLineage; source historical. |
| GIS-T021 | Merge | Merge retains source field history and lineage. |
| GIS-T022 | Lineage cycle | Attempted field lineage cycle rejected. |
| GIS-T023 | GIS dry run | Import dry-run creates no canonical field. |
| GIS-T024 | Partial import | VALID_ONLY mode commits valid features and reports invalid ones. |
| GIS-T025 | File hash | Committed feature traces to import file hash/feature id. |
| GIS-T026 | SHP encoding | Unsupported/ambiguous encoding surfaced; no silent mojibake commit. |
| GIS-T027 | GPKG | GeoPackage vectors import with CRS/layer metadata. |
| GIS-T028 | Offline field vectors | 1k cached fields available without network. |
| GIS-T029 | Offline map pack | Download pauses/resumes and survives app restart on supported mobile. |
| GIS-T030 | Offline scene date | Cached NDVI always displays capturedAt. |
| GIS-T031 | Layer security | Unauthorized field omitted from map/tile query. |
| GIS-T032 | Scene discovery | STAC query intersects field geometry/date and registers provider IDs. |
| GIS-T033 | Legacy endpoint isolation | Changing CDSE endpoint requires provider config, not domain/mobile schema change. |
| GIS-T034 | L2A provenance | Metric records Sentinel product processing level/baseline/scene. |
| GIS-T035 | NDVI formula | B08=.7, B04=.2 -> expected formula result within tolerance. |
| GIS-T036 | Cloud mask | Masked cloud/shadow pixels excluded from zonal stats. |
| GIS-T037 | Low coverage | Metric below valid-pixel threshold has LOW_COVERAGE and blocks configured critical alert. |
| GIS-T038 | Mixed resolution | NDMI/NDRE processing records resampling target/method. |
| GIS-T039 | No diagnosis | Low NDVI creates anomaly/inspection context, not automatic disease fact. |
| GIS-T040 | Time-series compatibility | Incompatible formula/profile series comparison is blocked or marked mixed basis. |
| GIS-T041 | Anomaly lineage | Anomaly links algorithm version and source MetricResults. |
| GIS-T042 | Scouting task | Anomaly -> task retains field/zone/scene refs. |
| GIS-T043 | Ground truth | Offline photo/GPS observation links back to anomaly after sync. |
| GIS-T044 | Anomaly dedupe | Same zone/condition in cooldown updates case instead of flooding. |
| GIS-T045 | Management zone version | New clustering run does not overwrite published prior zone. |
| GIS-T046 | Soil interpolation | Interpolated surface retains sample refs/method/version. |
| GIS-T047 | Yield cleaning | Raw yield points preserved; cleaned map references cleaning profile. |
| GIS-T048 | Prescription approval | Vendor export impossible before required agronomist approval. |
| GIS-T049 | Prescription portability | Changing machine adapter does not change canonical prescription geometry/rates. |
| GIS-T050 | Overlap/skip | Coverage analytics correctly compute union/overlap/uncovered against target. |
| GIS-T051 | Raster memory | Field processing reads windows/chunks; worker memory does not scale to full scene. |
| GIS-T052 | Raster worker failure | Failed worker retries/idempotently without duplicate MetricResult. |
| GIS-T053 | Provider outage | Saved scenes/metrics remain readable, new jobs show provider error. |
| GIS-T054 | Provider swap | Mock alternate provider passes same scene/metric contract. |
| GIS-T055 | Large bbox abuse | Oversized bbox/feature request rejected/capped. |
| GIS-T056 | Large geometry NFR | 100k-vertex path bounded and does not block API worker pool. |
| GIS-T057 | Area delta | Geometry approval above configured delta requires warning/reason. |
| GIS-T058 | Audit | Split/merge/geometry approval/prescription publish create audit records. |
| GIS-T059 | PII GPS | User without employee-location permission cannot access linked personal tracks. |
| GIS-T060 | Cloud replica | LocalPrimary cloud copy cannot edit canonical Field geometry. |

# 23. Codex implementation order

| Stage | Component | Deliverable |
| --- | --- | --- |
| G24-01 | PostGIS spatial primitives | Typed geometries, indexes, CRS/unit helpers, geometry validation. |
| G24-02 | Field/LandPlot model | Snapshots, allocations, geometry versions, lineage. |
| G24-03 | Geometry commands | Create/edit/approve/split/merge/compare. |
| G24-04 | GIS import/export | GeoJSON/KML/GPKG/SHP/WKT staging/commit. |
| G24-05 | Map query/read models | Viewport APIs, simplification, layer permissions. |
| G24-06 | Flutter offline maps | Field vectors + offline regions + cache metadata. |
| G24-07 | Cadastre provider port | Capability model, snapshot, fallback flows; provider adapter only after verified access. |
| G24-08 | Remote scene catalog | STAC/provider scene search/metadata/assets. |
| G24-09 | Raster worker | Mask/index/zonal statistics + object storage. |
| G24-10 | Time-series/anomaly | Metric history, algorithms, anomaly geometry. |
| G24-11 | Scouting bridge | Anomaly -> notification/task/route/ground truth. |
| G24-12 | Agro maps | Soil/yield/application/prescription layers and vendor adapters. |
| G24-13 | Performance/security tests | NFR + scope + file/raster abuse tests. |

# 24. Verified current technical sources

| ID | Source | URL/file | Use |
| --- | --- | --- | --- |
| S01 | НСПД portal | https://www.nspd.gov.ru/ | Current public portal presence; API availability deliberately not assumed. |
| S02 | Copernicus CDSE STAC | https://documentation.dataspace.copernicus.eu/APIs/STAC.html | Current STAC v1 endpoint, collections/search/filter; legacy endpoint deprecated 17.11.2025. |
| S03 | Copernicus Sentinel-2 | https://documentation.dataspace.copernicus.eu/Data/SentinelMissions/Sentinel2.html | L2A surface reflectance, SCL, 13 bands, B04/B08 10m baseline, masking examples. |
| S04 | CDSE APIs | https://documentation.dataspace.copernicus.eu/APIs.html | STAC/OData/Sentinel Hub/openEO provider capabilities. |
| S05 | CDSE API path update 2026 | https://dataspace.copernicus.eu/news/2026-3-9-api-path-structure-updates-sentinel-hub-services | 2026 path evolution; adapter/versioning rationale. |
| S06 | MapLibre Flutter Advanced | https://maplibre.org/flutter-maplibre-gl/advanced/ | Offline regions, PMTiles, performance and map features. |
| S07 | MapLibre April/August 2026 | https://maplibre.org/news/2026-05-02-maplibre-newsletter-april-2026/ | flutter-maplibre-gl offline-region improvements/current maintenance. |
| S08 | PostGIS manual | https://postgis.net/docs/en/ | Spatial storage/functions and GiST-based spatial indexes. |
| S09 | RFC 7946 GeoJSON | https://www.rfc-editor.org/info/rfc7946/ | GeoJSON WGS84 longitude/latitude interchange semantics. |
| S10 | OGC GeoPackage 1.4 | https://www.ogc.org/standards/geopackage/ | Portable SQLite vector/raster/tile container, especially useful for mobile/limited connectivity. |
| S11 | Document №12 | document_12_offline_sync_specification.docx | Offline map/QR/GPS and LocalPrimary behavior. |
| S12 | Document №17 | document_17_kpi_formula_registry.docx | Canonical spectral index formulas/quality flags. |
| S13 | Document №20 | document_20_non_functional_requirements.docx | GIS/raster/mobile performance targets. |
| S14 | Document №23 | document_23_data_governance_master_data_source_of_truth.docx | Spatial source-of-truth/provenance rules. |

# 25. Fixed decisions

LandPlot, Field and FieldSeason are separate canonical entities.

A Field can combine multiple cadastral parcels and partial allocations; cadastral identity never substitutes Field ID.

Operational Field geometry is versioned and platform-governed; cadastral geometry is a reference/source snapshot unless explicitly adopted through workflow.

We do not promise a stable official Russian cadastral API until it is verified for the deployment; provider capability/fallback is first-class.

PostGIS is canonical spatial storage; EPSG:3857 is not used for authoritative area/distance calculation.

GeoJSON, GeoPackage, KML/KMZ, Shapefile and WKT are supported through staged validation/import policies.

MapLibre is the renderer abstraction; offline field vectors and selected map/satellite resources are first-class mobile data.

Copernicus Sentinel-2 L2A is the initial reference remote-sensing source, not a vendor lock-in.

NDVI baseline uses NIR/Red provider mapping; Sentinel-2 profile uses B08/B04. Other indices are versioned FormulaProfiles.

Cloud/shadow/no-data masking and validPixelPct are mandatory before agronomic anomaly decisions.

An index anomaly is not a diagnosis. The primary workflow is anomaly -> scouting/ground truth -> action -> later comparison.

Prescription/management-zone outputs require method/version and, by default, human agronomic approval before machinery export.

All major advanced GIS/remote sensing functions are marked as competitive differentiators but remain testable domain workflows rather than marketing-only features.

# 26. Acceptance Criteria Document №24

170 stable GIS-* requirements are specified.

60 spatial/remote-sensing acceptance tests are defined.

Codex can implement field creation by drawing/import/cadastral-assisted workflow without inventing cadastral API access.

LandPlot<->Field many-to-many allocation and geometry history are explicit.

PostGIS/CRS/area/index/import rules prevent common spatial correctness errors.

Offline maps remain consistent with the mobile sync architecture.

Satellite scene, cloud mask, index formula, coverage and anomaly provenance are explicit.

The satellite workflow produces actionable scouting tasks and ground-truth links.

The document can later become /gis/entities.yaml, /gis/provider-contracts.yaml, /gis/remote-sensing.yaml and acceptance fixtures in the Codex Implementation Pack.

Следующий документ №25 - IoT Contract. Он определит единый контракт между будущими датчиками/RetrofitNode/gateway и уже готовым приложением: device registry, measurements, commands, MQTT/API envelopes, quality/time/idempotency, RS485/Modbus/4-20mA/PNP-NPN adapters, offline edge buffering и security - без превращения конкретного железа в зависимость бизнес-логики.