ДОКУМЕНТ №25

# IoT Contract

Device/Edge/Telemetry Contract для Codex: sensors, RetrofitNode, gateways, MQTT, Modbus, LoRaWAN, OPC UA, RS-485, 4-20mA, PNP/NPN, pulse, GPS, scales, buffering, commands, security и business evidence

IoT принцип Железо является заменяемым источником наблюдений, а не фундаментом приложения. Телефон, импорт, API, сторонний трекер, собственный RetrofitNode и PLC-adapter должны приходить к одному canonical measurement/evidence contract. Это позволяет начать полностью software-first и подключать датчики позже без переписывания бизнес-модулей.

Версия 1.0 • 250 IoT requirements • 70 acceptance tests • 26 сентября 2026 г.

# 1. Scope и нормативная рамка

| Reference | Use in project |
| --- | --- |
| ГОСТ Р 70924-2023 | Действующая российская типовая архитектура IoT; терминология/архитектурные представления. |
| ГОСТ Р 71777-2024 | Действующие российские термины и определения IoT. |
| ГОСТ Р 71840-2024 | Действующие общие требования к платформе обмена данными для IoT services. |
| ISO/IEC 30141:2024 | Актуальная международная IoT reference architecture; российский ГОСТ 70924 основан на предыдущем ISO/IEC 30141:2018. |
| MQTT 5.0 OASIS Standard | IP publish/subscribe transport profile. |
| Modbus Application Protocol V1.1b3 + Serial Line Guide V1.02 | Register/device adapters. |
| LoRaWAN 1.0.4 / 1.1 | Optional LPWAN field transport profiles. |
| OPC UA 1.05.x | Optional industrial integration/data-access profile. |

Важно ГОСТ/ISO задают архитектурную рамку, но конкретные электрические схемы, Ex-защита, поверка средств измерений, EMC, питание и промышленная безопасность зависят от конкретного устройства и места установки. Этот документ не сертифицирует generic RetrofitNode для опасных зон.

# 2. Reference Architecture

Sensor / PLC / scale / GPS / weather station
     |
 RS-485 / Modbus / 4-20mA / PNP-NPN / pulse / vendor protocol
     |
 RetrofitNode / Edge Adapter / Gateway
     |
 local normalization + durable buffer
     |
 MQTT 5 / HTTPS batch / OPC UA adapter
     |
 Local Server IoT Ingest
     |
 Raw Measurement -> Normalized Measurement -> Evidence Mapping
     |
 Domain Command / Notification / Analytics
     |
 Edge-to-Cloud replication (optional, asynchronous)

# 3. Canonical IoT entities

| Entity | Purpose | Key fields |
| --- | --- | --- |
| IoTDevice | Canonical end-device/virtual device. | id, tenantId, deviceType, manufacturer, model, serial?, lifecycle, capabilities, firmwareVersion |
| EdgeGateway | Gateway/edge software/hardware. | id, siteId, hostname, version, lastSeenAt, authorityServerId, health |
| DeviceInstallation | Effective binding/install context. | deviceId, asset/field/site ref, installedAt, removedAt, position, notes |
| DeviceChannel | Physical/logical input/output channel. | deviceId, channelCode, ioType, address, direction, signalDefinitionId |
| SignalDefinition | Semantic signal contract. | code, quantityKind, canonicalUnit, measurementType, ranges, aggregationPolicy |
| DeviceCapability | Supported protocol/feature/command. | deviceType/deviceId, capabilityCode, parameters |
| MeasurementRaw | Immutable raw observation. | messageId, deviceId, channelId, rawValue/rawPayloadRef, timestamps, protocolMeta |
| MeasurementNormalized | Scaled/calibrated canonical observation. | rawId, canonicalValue, unit, quality, transformProfileId |
| MeasurementBatch | Batch ingest receipt. | batchId, gatewayId, createdAt, receivedAt, counts, status |
| TelemetryMappingProfile | Decoder/register/channel mapping. | version, protocol, source addressing, transforms, effective dates |
| CalibrationProfile | Calibration/scaling metadata. | channelId, method, coefficients/table, validFrom/to, evidence |
| DeviceCredential | Secret reference only. | device/gateway, credentialType, secretRef, status, expiresAt |
| MQTTProfile | Broker/topic/QoS/session policy. | brokerRef, topicTemplate, qos, retainPolicy, sessionExpiry |
| ModbusMap | Unit/register map. | transport, unitId, function, address, dataType, byteOrder, pollMs |
| LoRaWANProfile | Network/application integration. | networkServer, devEUIRef, joinEUIRef, fPort/decoder, regionProfile |
| OPCUAMap | OPC node mapping. | serverRef, nodeId, attribute, sampling/subscription, securityProfile |
| DeviceCommand | Requested actuator/config action. | id, target, type, params, requestedBy, expiresAt, state, idempotencyClass |
| DeviceCommandResult | Delivery/execution result. | commandId, state, timestamps, resultCode, payloadRef |
| EdgeBufferStatus | Store-forward state. | gatewayId, pendingCount, bytes, oldestAge, drops, lastAckCursor |
| DeviceHealthSnapshot | Health projection. | deviceId, lastSeen, connectivity, battery, link, error counters, calibration status |
| FirmwareArtifact | Approved firmware metadata. | deviceType, version, checksum, signatureRef?, objectRef, status |
| FirmwareRollout | Staged rollout. | artifactId, cohort, state, success/failure counts, pause reason |
| DeviceConfigDesired | Versioned desired configuration. | target, configVersion, checksum, payloadRef |
| DeviceConfigReported | Reported applied config. | target, configVersion, checksum, observedAt, status |
| BusinessEvidenceLink | Measurement/event -> domain object. | evidenceRef, domainEntityRef, mappingProfileId, acceptedAt, role |

# 4. Measurement Envelope

measurement:
  schemaVersion: 1
  messageId: uuid
  tenantId: uuid
  deviceId: uuid
  channelId: uuid
  gatewayId: uuid?
  deviceSequence: 84532?
  measurementType: ANALOG | STATE | EVENT | COUNTER | POSITION | WEIGHT
  observedAt: 2026-09-26T10:00:00Z
  gatewayReceivedAt: 2026-09-26T10:00:01Z?
  receivedAt: server assigned
  raw:
    value: 12.483
    unit: mA?
    payloadRef: optional
  normalized:
    value: 4.21
    unit: bar
    transformProfileId: uuid
  quality:
    code: GOOD
    clockQuality: TIME_SYNCED
  protocol:
    type: MODBUS_RTU
    metadata: {...}

# 5. Raw -> Normalized -> Business Evidence

RAW
  immutable register/frame/ADC/GPS/device payload
        |
 decoder / scaling / calibration / unit conversion
        v
NORMALIZED MEASUREMENT
  canonical unit + quality + transform version
        |
 EvidenceMappingProfile / threshold / state derivation
        v
BUSINESS EVIDENCE
        |
 explicit domain command/rule/authorized workflow
        v
BUSINESS FACT

Почему так Если датчик сломан, зашумлён, откалиброван неправильно или поставщик поменял формат, мы можем пересчитать normalized layer и расследовать источник. Если же датчик напрямую изменял склад, простой или выпуск, восстановить правильную историю было бы намного сложнее.

# 6. Physical / electrical input profiles

| Interface | Canonical edge behavior | Notes |
| --- | --- | --- |
| RS-485 | Transport only; device protocol/profile explicit. | Often Modbus RTU; no native confidentiality/authentication assumption. |
| 4-20 mA | Measure raw current -> line diagnostics -> engineering conversion. | Pressure/level/flow/temp transmitters. |
| 0-10 V | Optional analog profile. | Only when hardware capability explicitly supports it. |
| PNP/NPN 24V | Typed digital input with wiring profile. | Machine RUN/STOP, limit switch, presence. |
| Dry contact | NO/NC + debounce. | Simple switches/relays. |
| Pulse | Counter/edge/debounce/factor/rollover. | Flow meters, RPM, production count. |
| Ethernet/TCP | Modbus TCP/vendor HTTPS/MQTT/OPC UA. | Factory baseline preferred over radio where feasible. |
| LoRa/LoRaWAN | Low-bandwidth field transport. | Battery/remote sensors; constrained downlink. |
| BLE/Wi-Fi | Near-device commissioning/local bridge optional. | Phone/gateway-assisted workflows. |

# 7. MQTT Contract

Topics (example, versioned):
agro/v1/{tenantId}/{siteId}/devices/{deviceId}/measurements
agro/v1/{tenantId}/{siteId}/devices/{deviceId}/state
agro/v1/{tenantId}/{siteId}/devices/{deviceId}/commands
agro/v1/{tenantId}/{siteId}/devices/{deviceId}/command-results
agro/v1/{tenantId}/{siteId}/gateways/{gatewayId}/health

| Policy | Decision |
| --- | --- |
| Version | MQTT 5.0 preferred; transport adapter can support 3.1.1 if required. |
| Telemetry QoS | QoS 1 default; QoS 0 only for explicitly lossy high-rate data. |
| Exactly once | Never rely on MQTT QoS alone for business exactly-once; messageId/idempotency always used. |
| Retain | Off for telemetry; allowed for explicit current state/config topics. |
| Security | TLS + per-device/gateway auth + topic ACL. |
| Payload | Canonical schema version independent from transport protocol. |
| LWT | Health hint only; device availability derived from broader heartbeat/data profile. |

MQTT 5.0 is an OASIS Standard and defines a lightweight publish/subscribe protocol for M2M/IoT; it includes QoS modes, but our business idempotency remains application-level.

# 8. HTTPS batch ingest

POST /v1/iot/measurements:batch
Idempotency-Key: <batch-id>

{
  "schemaVersion": 1,
  "gatewayId": "...",
  "items": [ ... measurement envelopes ... ]
}

202/200:
{
  "accepted": 98,
  "duplicates": 1,
  "rejected": 1,
  "results": [...]
}

# 9. Modbus Contract

modbus_map:
  version: 3
  transport: RTU
  endpoint:
    serialPort: /dev/ttyUSB0
    baud: 9600
    parity: EVEN
    stopBits: 1
  unitId: 7
  signals:
    - channel: pressure
      function: HOLDING_REGISTER
      address: 40010
      count: 2
      dataType: FLOAT32
      byteOrder: BIG
      wordOrder: SWAP
      pollMs: 1000
      transformProfile: pressure_v2

Modbus Organization still publishes Application Protocol V1.1b3 as the Modbus application protocol specification, while its site explicitly marks the old 1996 serial specification as legacy and directs new serial implementations to the Serial Line Protocol and Implementation Guide V1.02.

# 10. LoRaWAN profile

| Aspect | Contract |
| --- | --- |
| Versions | Network-server profiles can support LoRaWAN 1.0.4 and 1.1; key/counter behavior never mixed. |
| Identifiers | DevEUI/JoinEUI stored as external protocol IDs. |
| Region | Regional parameters configured by approved network-server/operator profile; RU deployment does not hard-code arbitrary frequencies in business app. |
| Payload | FPort/codec decoder versioned. |
| Keys | Root/session keys remain secret/network-server concern. |
| Uplink | Decoded into canonical MeasurementEnvelope. |
| Downlink | Rate/duty-cycle/battery constraints respected; not a general low-latency actuator channel. |

LoRa Alliance publishes the LoRaWAN 1.0.4 package and LoRaWAN 1.1 specification; 1.0.4 reference implementation information includes RU864 among validated regions. Our contract stays network-server/provider neutral.

# 11. OPC UA industrial profile

| Aspect | Rule |
| --- | --- |
| Version | Pin tested OPC UA 1.05.x feature set/profile. |
| Mapping | NodeId/attribute -> SignalDefinition via versioned OPCUAMap. |
| Data | SourceTimestamp/ServerTimestamp and StatusCode retained. |
| Quality | Bad/Uncertain status propagates to canonical quality. |
| Security | Certificates/trust/security policies explicitly configured. |
| Writes/methods | Disabled by default; DeviceCommand profile required. |

OPC Foundation's current online 1.05 series is actively revised; several core parts are at 1.05.07 in 2026. Therefore we pin tested capabilities rather than storing one vague 'OPC UA version' string.

# 12. Scale / GPS / machine-state adapters

| Adapter | Evidence produced | Business boundary |
| --- | --- | --- |
| Weight terminal | WeightReading: value/type/stable/device time/raw ref. | Weighing workflow chooses readings and closes weighing. |
| GPS tracker | Position/track with accuracy/speed/heading. | Can detect route/geofence/work evidence; cannot alone close task. |
| Machine digital state | RUN/STOP/IDLE transition evidence. | Downtime interval may derive; reason remains business workflow. |
| Fuel level/flow | Level/volume/rate/pulses. | Can create anomaly/refuel candidate; stock/fuel ledger is separate. |
| Production counter | Counter/pulse output evidence. | Can support quantity; production order close/quality not automatic. |

# 13. Edge offline buffer

COLLECT
  -> write durable edge queue
  -> local ACK from edge acquisition
  -> try Local Server
      success -> authoritative ACK cursor -> compact
      unavailable -> retain
  -> retry/backoff/batching
  -> overflow policy by priority

| Priority | Examples | Overflow preference |
| --- | --- | --- |
| P0 critical | Alarm/state transition/weight event/command result | Preserve; raise storage alarm early. |
| P1 operational | GPS/work state/production counters | Preserve longer; downsample only by explicit profile. |
| P2 periodic | Temperature/soil/weather routine samples | Can downsample/drop oldest under declared emergency policy. |
| P3 media | Images/video | Separate queue; must never block P0/P1 telemetry. |

# 14. Device Commands & Actuation

DeviceCommand:
  id
  tenantId
  targetDeviceId
  capabilityCode
  parameters
  requestedBy / automationRuleId?
  requestedAt
  expiresAt
  idempotencyClass: IDEMPOTENT | NON_IDEMPOTENT
  state
  correlationId

Safety boundary Команда приложения не является emergency stop и не заменяет локальную защиту, safety PLC, реле безопасности, блокировки или требования промышленной безопасности. Любой actuator use-case проходит отдельную risk review. V1 по умолчанию read-only monitoring.

# 15. Device lifecycle / commissioning

NEW
 -> COMMISSIONING
 -> ACTIVE
 -> DEGRADED
 -> MAINTENANCE
 -> ACTIVE
 -> SUSPENDED
 -> RETIRED

Any state -> REVOKED (security)

| Commission step | Evidence |
| --- | --- |
| Register hardware | Device canonical ID + external identifiers. |
| Provision credential | SecretRef/certificate status. |
| Declare capabilities | Protocols/channels/commands. |
| Bind channels | SignalDefinition + mapping version. |
| Install/bind asset/site | DeviceInstallation. |
| Connectivity test | Health + time sync. |
| Known-value test | Measurement/quality/calibration validation. |
| Activate | Audit event + effective mapping version. |

# 16. Security model

| Layer | Controls |
| --- | --- |
| Device/gateway identity | Per-device credential, revoke/rotate, optional mTLS. |
| MQTT | TLS, client identity, topic ACL, broker internal/LAN scope. |
| Legacy serial/Modbus | Network segmentation, gateway isolation, no direct external exposure. |
| LoRaWAN | Root/session keys protected by network/join-server profile. |
| OPC UA | Certificates/trust lists/security mode. |
| Commands | Separate permission/allowlist/expiry/audit. |
| Firmware | Approved artifact/checksum/signature where supported. |
| Secrets | Never normal logs/API/support bundle. |

# 17. Firmware and configuration

Desired and reported configuration are separate states.

Mapping/edge configuration has version + checksum.

OTA rollout is cohort-based and pausable; constrained battery radio devices may not support OTA.

FirmwareArtifact is an approved immutable artifact reference, not arbitrary URL.

Failed configuration/firmware update never silently marks device current.

# 18. Device health and diagnostics

| Signal | Use |
| --- | --- |
| lastSeenAt | Connectivity freshness. |
| heartbeat age | Derived online/offline/degraded state. |
| battery/voltage | Maintenance planning. |
| RSSI/SNR | Radio link quality. |
| gateway buffer backlog | Connectivity/storage risk. |
| decode/CRC/timeout errors | Protocol health. |
| clock skew | Timestamp trust. |
| calibration due | Measurement quality. |
| firmware/config drift | Configuration compliance. |

# 19. Data quality rules

| Rule class | Example |
| --- | --- |
| Range | Pressure <0 or >rated limit -> OUT_OF_RANGE. |
| Rate-of-change | Tank level jump impossible for configured max flow -> UNCERTAIN/anomaly. |
| Gap | No expected sample for 15 min -> missing-data health issue. |
| Stuck | Same value beyond context-specific duration -> potential stuck sensor. |
| Cross-sensor | Two redundant sensors diverge > tolerance -> issue; originals retained. |
| Calibration | Expired calibration -> quality downgraded according to policy. |
| Clock | Time skew > threshold -> TIME_UNCERTAIN. |
| Decoder | Payload cannot decode -> BAD + raw payload diagnostic. |

# 20. Business Evidence Mapping

evidence_mapping:
  code: MACHINE_STOP_V1
  signal: MACHINE_STATE
  condition:
    equals: STOP
  produce:
    evidenceType: MACHINE_STOP_SIGNAL
  domainAction:
    mode: PROPOSE
    command: OPEN_DOWNTIME_CANDIDATE
  autoReason: false
  minQuality: GOOD
  version: 1

Это ключевой слой между IoT и бизнес-моделью: hardware changes do not rewrite production/warehouse/weighing modules.

# 21. Storage / retention

| Data | Default strategy |
| --- | --- |
| Raw high-rate telemetry | Partitioned/time-retained; configurable TTL/downsampling. |
| Normalized telemetry | Longer retention according to use; partitioned. |
| Weight/device evidence used in closed business fact | Retained per domain fact/document policy. |
| Command/result | Long-term operational/audit retention. |
| Health metrics | Operational retention + aggregated history. |
| Media | Object store lifecycle; separate from telemetry DB. |
| Cloud replica | May receive filtered/downsampled telemetry rather than all raw samples. |

# 22. API endpoints

| Method | Path | operationId | Purpose |
| --- | --- | --- | --- |
| GET/POST | /v1/iot/devices | iotDeviceList/Create | Device registry. |
| GET/PATCH | /v1/iot/devices/{id} | iotDeviceGet/Update | Metadata/lifecycle allowed fields. |
| POST | /v1/iot/devices/{id}/commands/commission | iotDeviceCommission | Commission workflow. |
| POST | /v1/iot/devices/{id}/commands/revoke | iotDeviceRevoke | Security revoke. |
| GET/POST | /v1/iot/channels | iotChannelList/Create | Channels. |
| GET/POST | /v1/iot/mapping-profiles | iotMappingProfileList/Create | Protocol/decoder mappings. |
| POST | /v1/iot/measurements:batch | iotMeasurementBatchIngest | Batch ingest. |
| GET | /v1/iot/telemetry | iotTelemetryQuery | Bounded/query/downsample telemetry. |
| POST | /v1/iot/device-commands | iotDeviceCommandCreate | Authorized command. |
| GET | /v1/iot/device-commands/{id} | iotDeviceCommandGet | State/result. |
| POST | /v1/iot/credentials | iotCredentialIssue | Provision. |
| POST | /v1/iot/credentials/{id}/commands/rotate | iotCredentialRotate | Rotate. |
| POST | /v1/iot/firmware-rollouts | iotFirmwareRolloutCreate | Staged rollout. |
| GET | /v1/iot/health | iotHealthQuery | Fleet/gateway health. |

# 23. Full IoT Requirements Registry

| Area | Count |
| --- | --- |
| API | 10 |
| ARCHITECTURE | 15 |
| BUSINESS_MAPPING | 10 |
| CALIBRATION | 10 |
| COMMANDS | 12 |
| DATA_QUALITY | 10 |
| DEVICE_MODEL | 15 |
| EDGE_BUFFER | 14 |
| ELECTRICAL_IO | 10 |
| FIRMWARE_CONFIG | 8 |
| GPS | 8 |
| HEALTH | 10 |
| HTTP_INGEST | 8 |
| LORAWAN | 10 |
| MEASUREMENT | 20 |
| MODBUS | 14 |
| MQTT | 14 |
| OPC_UA | 8 |
| PERFORMANCE | 6 |
| SECURITY | 14 |
| STORAGE | 8 |
| TIME | 8 |
| WEIGH_DEVICE | 8 |
| TOTAL | 250 |

## 23.1. API

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-235 | Device registry API | CRUD/lifecycle under IoT admin permissions. | MUST | ALL |
| IOT-236 | Channel/signal API | Typed configuration and mapping endpoints. | MUST | ALL |
| IOT-237 | Batch ingest API | POST /v1/iot/measurements:batch. | MUST | ALL |
| IOT-238 | Gateway health API | Heartbeat/config/backlog metadata. | MUST | ALL |
| IOT-239 | Command API | Create/query/cancel allowed DeviceCommand. | MUST | ALL |
| IOT-240 | Provisioning API | One-time/bootstrap enrollment separated from normal ingest. | MUST | ALL |
| IOT-241 | Credential API | Issue/rotate/revoke; secret material shown only at creation where applicable. | MUST | ALL |
| IOT-242 | Firmware/config API | Inventory/desired/reported/rollout. | MUST | ALL |
| IOT-243 | Telemetry query API | Downsampled/default query with bounded time range and pagination. | MUST | ALL |
| IOT-244 | Raw diagnostic API restricted | Raw frame/payload access privileged and retention-limited. | MUST | ALL |

## 23.2. ARCHITECTURE

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-001 | IoT is optional acquisition layer | Core business workflows work with human/import/API sources without hardware. | MUST | ALL |
| IOT-002 | No direct device-to-database writes | All device data enters through Edge/Ingest contracts. | MUST | ALL |
| IOT-003 | Raw measurement != business fact | Device observations remain evidence until domain mapping/command accepts them. | MUST | ALL |
| IOT-004 | Vendor-neutral canonical model | Canonical Device/Signal/Measurement schemas do not contain vendor-specific business semantics. | MUST | ALL |
| IOT-005 | Local-first edge | Gateway can acquire/buffer data with external Internet unavailable. | MUST | ALL |
| IOT-006 | LocalPrimary alignment | In LOCAL_PRIMARY, IoT data is ingested by local authority first. | MUST | ALL |
| IOT-007 | Cloud not required for field/factory ingest | Cloud replication is downstream and asynchronous. | MUST | ALL |
| IOT-008 | Protocol adapters isolated | Modbus/MQTT/LoRaWAN/OPC UA/vendor protocols live behind adapters. | MUST | ALL |
| IOT-009 | At-least-once transport safe | Business effects are idempotent even if transport duplicates messages. | MUST | ALL |
| IOT-010 | No safety PLC replacement | V1 IoT platform is monitoring/operational evidence, not a safety-certified control system. | MUST | ALL |
| IOT-011 | Separate telemetry and command planes | Read/measurement path and actuator command path use separate permissions/policies. | MUST | ALL |
| IOT-012 | Edge failure isolation | Bad device/protocol cannot block core API or other device adapters. | MUST | ALL |
| IOT-013 | Schema versioning | Every message envelope includes schema version. | MUST | ALL |
| IOT-014 | Source provenance mandatory | Every measurement identifies source device/channel/gateway. | MUST | ALL |
| IOT-015 | Edge bridge has no domain DB access | Bridge talks to ingest/command API or broker only. | MUST | ALL |

## 23.3. BUSINESS_MAPPING

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-217 | Evidence mapping profile | Signal -> candidate business event mapping is explicit/versioned. | MUST | ALL |
| IOT-218 | Weight evidence | WeightReading may be selected into Weighing; raw reading itself not automatically a closed weighing. | MUST | ALL |
| IOT-219 | Machine state evidence | RUN/STOP signal may derive Downtime interval; final reason separate. | MUST | ALL |
| IOT-220 | Fuel evidence | Level/flow/pulse can propose refuel/drain anomaly; ledger movement requires domain workflow. | MUST | ALL |
| IOT-221 | Production count evidence | Pulse/counter can support output count but production close/quality remains workflow. | MUST | ALL |
| IOT-222 | GPS evidence | Track supports area/route/work detection but does not directly close operation. | MUST | ALL |
| IOT-223 | Environmental evidence | Weather/soil sensors become observations/alerts according to profile. | MUST | ALL |
| IOT-224 | Threshold alert | Threshold crossing may create NotificationCase without altering underlying business entity. | MUST | ALL |
| IOT-225 | Automatic command threshold | Any future automatic actuation requires separate AutomationRule with risk/permission/safety policy. | MUST | ALL |
| IOT-226 | Mapping version retained | Business fact/event references evidence mapping version. | MUST | ALL |

## 23.4. CALIBRATION

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-059 | Engineering conversion versioned | raw -> engineering conversion identified by Mapping/Calibration version. | MUST | ALL |
| IOT-060 | Linear scaling supported | y = a*x + b. | MUST | ALL |
| IOT-061 | Piecewise/table conversion supported | For tank level/flow/irregular sensor curves. | MUST | ALL |
| IOT-062 | Unit conversion centralized | Use Unit/Formula registry, not adapter-specific ad hoc arithmetic. | MUST | ALL |
| IOT-063 | Calibration effective dates | Historical values retain calibration used at observation time. | MUST | ALL |
| IOT-064 | Calibration expiry alert | Channels requiring periodic calibration have due/overdue status. | MUST | ALL |
| IOT-065 | Manual calibration requires audit | Actor/reference instrument/reason/evidence recorded. | MUST | ALL |
| IOT-066 | No silent retroactive recalculation | Reprocess historical normalized values only as new version/run. | MUST | ALL |
| IOT-067 | Physical plausible ranges | Per-channel min/max used as quality flag, not automatic clipping. | MUST | ALL |
| IOT-068 | Deadband/filter versioned | Filtering/debounce parameters part of signal mapping profile. | MUST | ALL |

## 23.5. COMMANDS

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-163 | Command object distinct from measurement | DeviceCommand has actor/rule/target/type/parameters/expiry/idempotency. | MUST | ALL |
| IOT-164 | Actuation disabled by default | Read-only monitoring is baseline deployment mode. | MUST | ALL |
| IOT-165 | Command permission | Separate DEVICE_COMMAND permission and target scope required. | MUST | ALL |
| IOT-166 | Command allowlist | Only registered DeviceCapability commands can be requested. | MUST | ALL |
| IOT-167 | Expiry/deadline | Stale command must not execute after business context expired. | MUST | ALL |
| IOT-168 | Command acknowledgment phases | QUEUED, DELIVERED, ACCEPTED_BY_DEVICE, EXECUTED, FAILED, EXPIRED, UNKNOWN. | MUST | ALL |
| IOT-169 | Execution result evidence | Result includes device/gateway timestamps and result payload/diagnostic. | MUST | ALL |
| IOT-170 | Idempotency | Retry of same commandId must not repeat dangerous effect if adapter/device can guarantee; otherwise command is NON_IDEMPOTENT and retry policy restricted. | MUST | ALL |
| IOT-171 | Safety interlock respected | Platform never bypasses PLC/local safety interlocks. | MUST | ALL |
| IOT-172 | No autonomous AI actuation baseline | AI may recommend/draft; direct actuation requires future explicit AutomationPolicy. | MUST | ALL |
| IOT-173 | Offline command policy | Commands requiring current state generally not queued indefinitely; expiry/profile enforced. | MUST | ALL |
| IOT-174 | Emergency stop out of scope | Safety E-stop remains dedicated certified/local circuitry/control, not app command. | MUST | ALL |

## 23.6. DATA_QUALITY

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-207 | Decoder failures retained diagnostically | Raw payload/hash and error code available per retention policy. | MUST | ALL |
| IOT-208 | Plausibility flags | Out-of-range value is flagged, not silently clipped. | MUST | ALL |
| IOT-209 | Rate-of-change rules optional | Impossible jumps create UNCERTAIN/issue. | MUST | ALL |
| IOT-210 | Cross-sensor validation optional | Redundant sensors can compare without deleting originals. | MUST | ALL |
| IOT-211 | Missing interval is explicit | No interpolation presented as measured value without estimated flag. | MUST | ALL |
| IOT-212 | Interpolation versioned | Derived/interpolated stream records algorithm/profile. | MUST | ALL |
| IOT-213 | Business mapping threshold | Only GOOD/allowed UNCERTAIN values can auto-trigger selected domain rules. | MUST | ALL |
| IOT-214 | Calibration quality affects downstream | Expired/bad calibration propagates quality flags. | MUST | ALL |
| IOT-215 | Sample aggregation preserves coverage | Average/min/max records n/time coverage and quality. | MUST | ALL |
| IOT-216 | Quality issue integration | Systemic device/channel quality creates DataQualityIssue/DeviceHealthIssue. | MUST | ALL |

## 23.7. DEVICE_MODEL

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-016 | Canonical device ID | Platform UUIDv7 is canonical; serial/MAC/DevEUI are attributes/external IDs. | MUST | ALL |
| IOT-017 | Gateway separate from end device | Gateway lifecycle/credentials/health modeled independently. | MUST | ALL |
| IOT-018 | Device channel modeled explicitly | Each measurable input/output has Channel identity and semantics. | MUST | ALL |
| IOT-019 | Signal definition separate from channel | Channel binds physical address to governed SignalDefinition. | MUST | ALL |
| IOT-020 | Asset binding versioned | Device/channel may bind to Vehicle/Asset/Field/Tank/Line with effective dates. | MUST | ALL |
| IOT-021 | One channel one active semantic binding | Ambiguous simultaneous business bindings prohibited unless profile explicitly multi-casts. | MUST | ALL |
| IOT-022 | Device type taxonomy | SENSOR, GATEWAY, RETROFIT_NODE, PLC_ADAPTER, SCALE_ADAPTER, GPS_TRACKER, WEATHER_STATION, CAMERA_ADAPTER, VIRTUAL_DEVICE. | MUST | ALL |
| IOT-023 | Lifecycle states | NEW, COMMISSIONING, ACTIVE, DEGRADED, MAINTENANCE, SUSPENDED, RETIRED, REVOKED. | MUST | ALL |
| IOT-024 | No serial-number uniqueness globally | Uniqueness scoped by manufacturer/profile/tenant where needed. | MUST | ALL |
| IOT-025 | Installation identity | Installation record stores location/asset/mounting/commission date. | MUST | ALL |
| IOT-026 | Calibration profile | Calibration/version/effective date linked to affected channels. | MUST | ALL |
| IOT-027 | Firmware metadata | Firmware version/build/hardware revision tracked where device supports it. | MUST | ALL |
| IOT-028 | Capabilities machine-readable | Supported transports, inputs, outputs, commands, sampling limits declared. | MUST | ALL |
| IOT-029 | Tags not semantics | Free tags aid search but cannot replace typed capability/channel metadata. | MUST | ALL |
| IOT-030 | Retired device history preserved | Historical measurements remain resolvable after device retirement. | MUST | ALL |

## 23.8. EDGE_BUFFER

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-069 | Store-and-forward | Gateway/edge buffers measurements during uplink outage. | MUST | ALL |
| IOT-070 | Durable local queue | Acknowledged local collection survives edge process restart/power loss within hardware limits. | MUST | ALL |
| IOT-071 | Oldest-first default replay | Backlog replay preserves per-device sequence where possible. | MUST | ALL |
| IOT-072 | Priority classes | Critical alarms/state changes can outrank routine high-frequency samples. | MUST | ALL |
| IOT-073 | Bounded storage | Retention/capacity policy prevents disk exhaustion. | MUST | ALL |
| IOT-074 | Overflow policy explicit | DROP_OLDEST_NONCRITICAL, DOWNSAMPLE, STOP_ACCEPTING, ALARM - never silent. | MUST | ALL |
| IOT-075 | Compression allowed | Buffered payload can compress without changing semantic identifiers. | MUST | ALL |
| IOT-076 | Batch upload | Replay uses bounded batches and backpressure. | MUST | ALL |
| IOT-077 | Ack cursor | Edge deletes/compacts only after authoritative ingest acknowledgement. | MUST | ALL |
| IOT-078 | Media separate | Camera/image payload buffer separate from critical telemetry queue. | MUST | ALL |
| IOT-079 | Backlog observability | count/bytes/oldestAge/dropCount visible. | MUST | ALL |
| IOT-080 | Configuration survives reboot | Device mapping/broker/server config durable and versioned. | MUST | ALL |
| IOT-081 | Config rollback | Invalid mapping/config can revert to last known good. | MUST | ALL |
| IOT-082 | Local LAN operation | Edge can deliver to Local Server without cloud DNS/Internet. | MUST | ALL |

## 23.9. ELECTRICAL_IO

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-119 | 4-20mA input profile | Raw current, engineering range, sensor fault thresholds and calibration explicit. | MUST | ALL |
| IOT-120 | 0-10V optional profile | Supported only by declared hardware capability with scaling profile. | MUST | ALL |
| IOT-121 | Digital PNP/NPN profile | Input electrical type/configuration explicit; software does not infer wiring. | MUST | ALL |
| IOT-122 | Dry contact profile | Debounce and normal-open/normal-closed semantics configured. | MUST | ALL |
| IOT-123 | Pulse/counter profile | Pulse factor, edge, debounce, counter width and rollover explicit. | MUST | ALL |
| IOT-124 | Frequency/RPM profile | Pulse frequency to RPM/flow conversion versioned. | MUST | ALL |
| IOT-125 | Sensor open/short diagnostics | If hardware supports diagnostics, emit quality/error signal. | MUST | ALL |
| IOT-126 | Isolation not assumed | Hardware installation profile records galvanic isolation/surge/protection capability. | MUST | ALL |
| IOT-127 | Power supply metadata | 24V/other supply requirement belongs to hardware profile, not canonical measurement schema. | MUST | ALL |
| IOT-128 | Ex-zone boundary | Potential explosive dust/gas areas require separately qualified/certified hardware; generic node not approved by software. | MUST | ALL |

## 23.10. FIRMWARE_CONFIG

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-189 | Config desired/reported states | Device config tracks desired vs reported version. | MUST | ALL |
| IOT-190 | Config version hash | Mapping/config payload has version/checksum. | MUST | ALL |
| IOT-191 | Atomic config apply | Device/gateway reports success/failure; partial apply explicit. | MUST | ALL |
| IOT-192 | Rollback | Last-known-good config retained where edge software supports. | MUST | ALL |
| IOT-193 | OTA staged rollout | Firmware rollout can target cohort/site/device type and pause on failures. | MUST | ALL |
| IOT-194 | Battery/radio constraints | OTA disabled/limited for constrained LoRaWAN devices unless suitable transport/profile. | MUST | ALL |
| IOT-195 | Firmware inventory | Current version/target/last update/status queryable. | MUST | ALL |
| IOT-196 | Configuration audit | Who changed mapping/poll interval/threshold tracked. | MUST | ALL |

## 23.11. GPS

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-129 | Canonical position fields | lat, lon, altitude?, speed?, heading?, accuracy?, observedAt, provider. | MUST | ALL |
| IOT-130 | Accuracy required when available | Position quality retained; low accuracy can be filtered by consumer policy. | MUST | ALL |
| IOT-131 | Track chunking | High-rate points grouped/chunked for efficient ingest/storage. | MUST | ALL |
| IOT-132 | No operation completion solely from proximity | Geofence/GPS can propose evidence/event; domain workflow decides completion. | MUST | ALL |
| IOT-133 | Impossible movement detection | Speed/jump plausibility flags quality instead of deleting raw point. | MUST | ALL |
| IOT-134 | GNSS time use | GNSS timestamp can improve clock quality and is recorded as source. | MUST | ALL |
| IOT-135 | Provider coexistence | Own tracker and external telematics provider can coexist with source metadata. | MUST | ALL |
| IOT-136 | Privacy linkage | Vehicle GPS becomes personal-data relevant when linked to employee, per privacy profile. | MUST | ALL |

## 23.12. HEALTH

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-197 | Last seen | Every active device/gateway exposes lastSeenAt. | MUST | ALL |
| IOT-198 | Connectivity state derived | ONLINE/OFFLINE/STale from heartbeat/message profile, not one transport disconnect only. | MUST | ALL |
| IOT-199 | Battery health | Voltage/percent/estimate preserved when available with quality. | MUST | ALL |
| IOT-200 | RSSI/SNR link metrics | Radio metrics retained when source supplies them. | MUST | ALL |
| IOT-201 | Communication error counters | Timeout/CRC/protocol/decoder errors observable. | MUST | ALL |
| IOT-202 | Gap detection | Expected-sampling channels can detect missing intervals. | MUST | ALL |
| IOT-203 | Sensor stuck detection optional | Constant-value anomaly creates health issue, not auto replacement of data. | MUST | ALL |
| IOT-204 | Drift/calibration due | Calibration/quality degradation alerts supported. | MUST | ALL |
| IOT-205 | Gateway resource health | CPU/RAM/disk/queue/backlog/time sync tracked for software gateway. | MUST | ALL |
| IOT-206 | Fleet health dashboard | Counts by ACTIVE/DEGRADED/OFFLINE/REVOKED and oldest backlog. | MUST | ALL |

## 23.13. HTTP_INGEST

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-097 | HTTPS batch ingest fallback | Gateways can POST measurement batches without MQTT. | MUST | ALL |
| IOT-098 | Idempotency header/key | Batch and item IDs dedupe retries. | MUST | ALL |
| IOT-099 | 202 for queued heavy processing | Ingest acknowledgement distinct from downstream business acceptance. | MUST | ALL |
| IOT-100 | Payload limits | Request count/bytes bounded; 413/actionable response. | MUST | ALL |
| IOT-101 | Compression | gzip/zstd where supported by client/server profile. | MUST | ALL |
| IOT-102 | mTLS/token auth | Gateway uses device/service credential, not user token. | MUST | ALL |
| IOT-103 | Partial batch result | Per-item accepted/duplicate/rejected reason returned. | MUST | ALL |
| IOT-104 | Retry-After respected | Server overload/provider maintenance conveys retry policy. | MUST | ALL |

## 23.14. LORAWAN

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-153 | LoRaWAN optional field transport | LoRaWAN is one transport option for low-bandwidth battery devices, not canonical domain. | MUST | ALL |
| IOT-154 | 1.0.4 and 1.1 profiles | Support network-server integration profiles as deployed; do not mix key/counter semantics. | MUST | ALL |
| IOT-155 | RU864 region profile | Russian deployment uses legal/approved regional parameters from network stack/operator profile. | MUST | ALL |
| IOT-156 | DevEUI/JoinEUI external identifiers | Stored as device attributes/external IDs, not platform primary key. | MUST | ALL |
| IOT-157 | Application payload decoder versioned | FPort/payload codec version recorded. | MUST | ALL |
| IOT-158 | Root/session keys protected | Never expose in ordinary API/log; preferably network/join-server custody. | MUST | ALL |
| IOT-159 | Frame counters not business counters | Used for protocol security/dedup, not measurement identity alone. | MUST | ALL |
| IOT-160 | Confirmed uplink used selectively | Avoid excessive radio/energy use; application-level missing-data policy remains. | MUST | ALL |
| IOT-161 | Downlink budget aware | Commands/config updates account for duty-cycle/network limits. | MUST | ALL |
| IOT-162 | Network-server adapter | ChirpStack/operator/vendor API/webhook can map into canonical ingest contract. | MUST | ALL |

## 23.15. MEASUREMENT

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-031 | Canonical measurement envelope | Message includes ids, timestamps, sequence, value, unit, quality, schemaVersion. | MUST | ALL |
| IOT-032 | messageId required | Globally/scoped unique message identifier supports deduplication. | MUST | ALL |
| IOT-033 | deviceSequence optional but recommended | Monotonic device/gateway sequence used to detect gaps/replays. | MUST | ALL |
| IOT-034 | observedAt distinct from receivedAt | Physical observation time and platform receipt time never conflated. | MUST | ALL |
| IOT-035 | gatewayReceivedAt optional | Preserve edge timing to diagnose radio/network delays. | MUST | ALL |
| IOT-036 | rawValue retained when transformation occurs | Engineering value never destroys original register/ADC/payload value. | MUST | ALL |
| IOT-037 | canonicalValue separate | Scaled/calibrated/unit-normalized value has transform version. | MUST | ALL |
| IOT-038 | unitCode mandatory for quantities | No unit inference from channel name. | MUST | ALL |
| IOT-039 | qualityCode mandatory | GOOD, UNCERTAIN, BAD, STALE, OUT_OF_RANGE, COMM_ERROR, SENSOR_ERROR, MANUAL_OVERRIDE etc. | MUST | ALL |
| IOT-040 | nullable value rules | Bad/comm-error observation can contain null canonical value with diagnostic metadata. | MUST | ALL |
| IOT-041 | location optional but typed | lat/lon/alt/accuracy/provider only where meaningful. | MUST | ALL |
| IOT-042 | batch ingestion supported | EnvelopeBatch carries many observations with bounded size. | MUST | ALL |
| IOT-043 | unknown fields tolerance versioned | Consumers follow schema compatibility policy; no guessing unknown semantic fields. | MUST | ALL |
| IOT-044 | payload hash optional | Raw payload checksum can prove ingest identity/debug. | MUST | ALL |
| IOT-045 | measurement immutable | Corrections/normalization revisions create new derived record/version, not overwrite raw. | MUST | ALL |
| IOT-046 | event vs sample distinction | Discrete event, counter, state and analog sample have explicit measurementType. | MUST | ALL |
| IOT-047 | state duration not inferred blindly | State changes can derive intervals only with gap/quality policy. | MUST | ALL |
| IOT-048 | counter rollover modeled | Counter width/rollover/reset semantics defined by channel profile. | MUST | ALL |
| IOT-049 | precision metadata | Scale/resolution/accuracy retained where source provides it. | MUST | ALL |
| IOT-050 | source trust class | Measurement records sourceTrust/validation profile for downstream policy. | MUST | ALL |

## 23.16. MODBUS

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-105 | Modbus TCP and Serial adapters | Support TCP and serial RTU through edge adapters. | MUST | ALL |
| IOT-106 | Use current application protocol semantics | Function codes/register types follow Modbus Application Protocol V1.1b3 profile. | MUST | ALL |
| IOT-107 | New serial implementation profile | Serial adapter follows current Modbus Serial Line implementation guide, not obsolete 1996 legacy spec. | MUST | ALL |
| IOT-108 | Register map configuration | Unit/slave, function, address, count, datatype, endian/word order explicit. | MUST | ALL |
| IOT-109 | No assumed register endian for 32/64-bit | Byte/word ordering must be mapping profile field. | MUST | ALL |
| IOT-110 | Polling interval bounded | Per mapping min interval prevents bus/device overload. | MUST | ALL |
| IOT-111 | Poll groups | Adjacent registers can batch-read where safe. | MUST | ALL |
| IOT-112 | Exception codes preserved | Protocol exception captured as diagnostic/quality. | MUST | ALL |
| IOT-113 | Timeout/retry bounded | No infinite hot polling on failed device. | MUST | ALL |
| IOT-114 | Write functions disabled by default | Actuation writes require command profile and explicit allowlist. | MUST | ALL |
| IOT-115 | Legacy serial compensation | RS-485/serial legacy networks treated untrusted/no native security; compensate with segmentation/gateway. | MUST | ALL |
| IOT-116 | Modbus Security optional | TLS/X.509 secured Modbus profile may be used where supported. | MUST | ALL |
| IOT-117 | Scaling outside register driver | Register driver returns typed raw value; business scaling/calibration governed separately. | MUST | ALL |
| IOT-118 | Device map versioning | Register map profile version retained in measurement provenance. | MUST | ALL |

## 23.17. MQTT

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-083 | MQTT 5.0 preferred IP transport | Use MQTT 5.0 features where broker/client support; MQTT 3.1.1 compatibility adapter optional. | MUST | ALL |
| IOT-084 | TLS transport | MQTT over TLS for production IP networks unless isolated legacy adapter terminates securely at edge. | MUST | ALL |
| IOT-085 | Client identity | Unique clientId per gateway/device session; no shared farm-wide client credentials. | MUST | ALL |
| IOT-086 | Topic namespace governed | Tenant/site/device hierarchy is fixed and versioned. | MUST | ALL |
| IOT-087 | QoS 1 telemetry baseline | At-least-once transport with application dedupe; QoS 0 allowed for explicitly lossy high-rate data. | MUST | ALL |
| IOT-088 | QoS 2 not business exactly-once | Application idempotency remains mandatory even if QoS 2 is used. | MUST | ALL |
| IOT-089 | Retained measurement disabled by default | Retained messages used only for explicit state/config topics. | MUST | ALL |
| IOT-090 | Last Will health | Gateway/device connection status may use LWT as health signal, not sole availability truth. | MUST | ALL |
| IOT-091 | Session expiry configured | Persistent-session policy bounded by deployment/profile. | MUST | ALL |
| IOT-092 | Topic ACL | Device publishes/subscribes only its authorized topic subset. | MUST | ALL |
| IOT-093 | Payload independent | Canonical JSON/CBOR/protobuf profile versioned separately from MQTT transport. | MUST | ALL |
| IOT-094 | No secrets in topics | Topics contain opaque IDs, not credentials/PII. | MUST | ALL |
| IOT-095 | Command response correlation | requestId/commandId links command and result. | MUST | ALL |
| IOT-096 | Broker isolation | Broker outage cannot corrupt canonical DB; edge buffers/retries. | MUST | ALL |

## 23.18. OPC_UA

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-145 | OPC UA optional industrial adapter | Use for PLC/SCADA/data-access integration where available. | MUST | ALL |
| IOT-146 | Current 1.05 family profile | Implementation must pin tested OPC UA 1.05.x capabilities, not vague 'OPC compatible'. | MUST | ALL |
| IOT-147 | NodeId mapping versioned | OPC node -> SignalDefinition mapping profile versioned. | MUST | ALL |
| IOT-148 | Subscription preferred for suitable signals | Polling vs subscriptions selected by source/server capability. | MUST | ALL |
| IOT-149 | Security modes explicit | Certificates/trust lists/security policy configured; no anonymous insecure production default. | MUST | ALL |
| IOT-150 | Server timestamp/source timestamp retained | Map into canonical time provenance. | MUST | ALL |
| IOT-151 | StatusCode mapped to quality | Bad/uncertain OPC status propagates to measurement quality. | MUST | ALL |
| IOT-152 | Method/write disabled by default | Commands require separate explicit actuator profile. | MUST | ALL |

## 23.19. PERFORMANCE

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-245 | Enterprise steady ingest | Meet Document 20: 5M normalized messages/day. | MUST | ALL |
| IOT-246 | Enterprise burst | Meet 2,000 msg/s for 5 min without durable loss. | MUST | ALL |
| IOT-247 | Ingest acknowledgement | P95 <=500 ms for gateway batch accepted into durable ingest queue. | MUST | ALL |
| IOT-248 | Normalized visibility | P95 <=5 s gateway receive -> queryable normalized telemetry under steady load. | MUST | ALL |
| IOT-249 | Critical local alert | P95 <=10 s local receive -> NotificationCase under normal load. | MUST | ALL |
| IOT-250 | Backpressure | Overload bounded; adapters/queues do not OOM core host. | MUST | ALL |

## 23.20. SECURITY

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-175 | Per-device/gateway identity | Unique revocable credential per installation/device class. | MUST | ALL |
| IOT-176 | Credential rotation | Rotation supported without changing canonical device ID. | MUST | ALL |
| IOT-177 | mTLS preferred for gateways | High-trust fixed gateways use client certificates where practical. | MUST | ALL |
| IOT-178 | No default shared password fleet-wide | Factory/default credentials must be changed/disabled. | MUST | ALL |
| IOT-179 | Least privilege topics/API | Credential restricts tenant/site/device and publish/subscribe paths. | MUST | ALL |
| IOT-180 | Secret values hidden | Keys/tokens/root keys never returned by normal GET API. | MUST | ALL |
| IOT-181 | Replay protection | Message IDs/sequences/protocol counters/timestamps used according to transport. | MUST | ALL |
| IOT-182 | Firmware authenticity | OTA firmware accepted only from approved artifact/profile; signed verification where hardware supports. | MUST | ALL |
| IOT-183 | Secure boot capability metadata | Tracked where hardware supports; not assumed universally. | MUST | ALL |
| IOT-184 | Industrial segmentation | Legacy insecure protocols isolated behind edge gateway/network controls. | MUST | ALL |
| IOT-185 | Unknown device quarantine | Unregistered/revoked device data rejected or quarantined, never canonical. | MUST | ALL |
| IOT-186 | Security audit | Commission/revoke/credential/config/command events audited. | MUST | ALL |
| IOT-187 | No public Redis/DB from IoT zone | Device network reaches only edge/ingest endpoints. | MUST | ALL |
| IOT-188 | Remote support bounded | No permanent vendor backdoor/account. | MUST | ALL |

## 23.21. STORAGE

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-227 | Raw telemetry retention configurable | High-frequency raw retention independent from canonical facts. | MUST | ALL |
| IOT-228 | Normalized telemetry partitioned | Time-series tables use partition/retention strategy before enterprise scale. | MUST | ALL |
| IOT-229 | No every-sample audit event | Operational audit references device/config/commands; telemetry itself is data, not generic audit spam. | MUST | ALL |
| IOT-230 | Downsampling jobs versioned | Minute/hour/day aggregates linked to raw/source window and method. | MUST | ALL |
| IOT-231 | Critical evidence retention stronger | Weight/production/legal evidence follows domain retention, not generic telemetry TTL. | MUST | ALL |
| IOT-232 | Object payload externalized | Images/large binary frames in S3/MinIO, not large DB blobs. | MUST | ALL |
| IOT-233 | Cloud replica optional | Local raw high-frequency telemetry need not replicate all samples; replication policy configurable. | MUST | ALL |
| IOT-234 | Deletion preserves lineage semantics | If raw TTL expires, derived fact indicates source class/expired raw reference where needed. | MUST | ALL |

## 23.22. TIME

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-051 | UTC canonical timestamps | All absolute timestamps stored UTC with source timezone if provided. | MUST | ALL |
| IOT-052 | Clock quality | TIME_SYNCED, TIME_UNCERTAIN, GATEWAY_TIMESTAMPED, SERVER_TIMESTAMPED. | MUST | ALL |
| IOT-053 | No wall-clock-only ordering | Idempotency/order uses messageId/sequence in addition to timestamps. | MUST | ALL |
| IOT-054 | Clock skew detection | Configured skew threshold creates device health/quality issue. | MUST | ALL |
| IOT-055 | Gateway time source monitored | NTP/chrony/GNSS time health exposed. | MUST | ALL |
| IOT-056 | Offline timestamp policy | Device observation time preserved after delayed upload. | MUST | ALL |
| IOT-057 | Missing device clock | Gateway/server timestamp allowed but quality explicitly lower. | MUST | ALL |
| IOT-058 | DST irrelevant to device storage | Business display timezone applied later. | MUST | ALL |

## 23.23. WEIGH_DEVICE

| ID | Requirement | Decision | Level | Profile |
| --- | --- | --- | --- | --- |
| IOT-137 | Scale/terminal adapter abstraction | Serial/TCP/Modbus/vendor API adapters produce common WeightReading. | MUST | ALL |
| IOT-138 | Stable flag | Reading includes stable/unstable if terminal exposes it. | MUST | ALL |
| IOT-139 | Raw instrument text/frame optional | Retain diagnostic frame/hash when allowed for audit. | MUST | ALL |
| IOT-140 | Resolution/division metadata | Instrument resolution retained when available. | MUST | ALL |
| IOT-141 | Gross/tare/net type explicit | No inference solely from sequence. | MUST | ALL |
| IOT-142 | Raw reading immutable | Business weighing references selected raw readings. | MUST | ALL |
| IOT-143 | Duplicate frame suppression | Adapter dedupes repeated identical terminal frames by profile without losing valid repeated weighings. | MUST | ALL |
| IOT-144 | Manual fallback | Weighing workflow remains usable with manual reading + reason/evidence if device unavailable. | MUST | ALL |

# 24. End-to-end IoT workflows

| ID | Scenario | Flow |
| --- | --- | --- |
| WF-IOT-01 | Commission RetrofitNode | Create device -> credential -> channels -> mapping -> install -> test measurement -> ACTIVE. |
| WF-IOT-02 | RS-485/Modbus sensor | Poll register -> raw -> typed raw -> scaling/calibration -> normalized measurement -> quality. |
| WF-IOT-03 | 4-20mA pressure sensor | ADC/current -> line fault check -> engineering conversion -> pressure -> threshold/anomaly. |
| WF-IOT-04 | PNP/NPN machine state | Digital transition -> debounce -> state evidence -> RUN/STOP interval candidate -> downtime workflow. |
| WF-IOT-05 | Pulse flow meter | Pulse counter -> rollover/delta -> volume/rate -> fuel/water evidence. |
| WF-IOT-06 | GPS tracker | Position batch -> quality/plausibility -> track -> geofence/work evidence. |
| WF-IOT-07 | Weight terminal | Frame -> raw WeightReading -> stable selection -> Weighing domain command. |
| WF-IOT-08 | LoRaWAN field sensor | Uplink -> network-server webhook -> decoder -> canonical envelope -> local/server ingest. |
| WF-IOT-09 | Internet outage edge buffer | Collect -> durable buffer -> priorities -> reconnect batch replay -> ACK -> compact. |
| WF-IOT-10 | Device replacement | Retire old -> install new -> preserve asset signal semantic identity -> mapping effective dates. |
| WF-IOT-11 | Calibration | Due -> calibration evidence -> publish profile -> new readings use new version; old preserved. |
| WF-IOT-12 | Firmware staged rollout | Approve artifact -> cohort -> update -> reported version -> pause/rollback on failure. |
| WF-IOT-13 | Device command | Authorize -> create command -> deliver -> device ack -> execute/result -> audit/expire. |
| WF-IOT-14 | Unknown device | Receive -> fail identity -> quarantine/reject -> alert admin; no canonical business effect. |
| WF-IOT-15 | Sensor data quality failure | Outlier/gap/stuck -> quality issue -> maintenance/scouting task -> resolve. |
| WF-IOT-16 | Cloud replication | Local canonical telemetry/business evidence -> policy aggregation/filter -> central replica. |

# 25. Mandatory Acceptance Tests

| Test | Area | Expected |
| --- | --- | --- |
| IOT-T001 | Core without hardware | Platform field/warehouse/weighing manual workflows operate with IoT module disabled. |
| IOT-T002 | Unknown device | Unregistered credential/data cannot create normalized/business fact. |
| IOT-T003 | Revoked device | Revoked device ingest rejected/quarantined. |
| IOT-T004 | Duplicate message | Same messageId twice yields one normalized effect. |
| IOT-T005 | Out-of-order | Late observedAt accepted while receivedAt/order metadata preserved. |
| IOT-T006 | Clock skew | Large device skew produces TIME_UNCERTAIN/health issue. |
| IOT-T007 | Raw immutable | Recalibration does not overwrite raw observation. |
| IOT-T008 | Calibration version | Historical normalized value references calibration active at observation. |
| IOT-T009 | Unit mapping | Unknown unit prevents silent canonical conversion. |
| IOT-T010 | 4-20mA conversion | Known raw current/profile gives expected engineering value. |
| IOT-T011 | 4-20mA fault | Configured under/over-range produces sensor fault quality. |
| IOT-T012 | Digital debounce | Contact bounce does not create multiple business state transitions. |
| IOT-T013 | Counter rollover | Rollover yields correct delta, no negative flow spike. |
| IOT-T014 | Modbus endian | Configured byte/word order decodes fixture correctly. |
| IOT-T015 | Modbus exception | Protocol exception yields diagnostic/quality, not zero reading. |
| IOT-T016 | Modbus timeout | Retries bounded and other devices continue polling. |
| IOT-T017 | Modbus write disabled | Unauthorized write register operation impossible. |
| IOT-T018 | MQTT QoS1 duplicate | Redelivery deduped by application messageId. |
| IOT-T019 | MQTT topic ACL | Device cannot publish another tenant/site topic. |
| IOT-T020 | MQTT retained | Telemetry topics not retained by default. |
| IOT-T021 | MQTT broker loss | Edge buffers measurements and later replays. |
| IOT-T022 | HTTP batch retry | Retried identical batch returns duplicate/accepted without duplicate facts. |
| IOT-T023 | Partial batch | One bad item does not ambiguously mark all items accepted. |
| IOT-T024 | Buffer power restart | Buffered acknowledged local data survives edge restart. |
| IOT-T025 | Buffer overflow | Configured policy emits drop/overflow metric and never silent loss. |
| IOT-T026 | Priority replay | Critical state event replays before bulk low-priority samples when policy configured. |
| IOT-T027 | Ack compaction | Edge does not delete buffered item before authoritative ACK. |
| IOT-T028 | GPS accuracy | Low-accuracy point flagged and consumer can exclude. |
| IOT-T029 | GPS impossible jump | Raw retained; normalized quality flagged. |
| IOT-T030 | GPS business boundary | Geofence arrival does not automatically close task. |
| IOT-T031 | Weight stable flag | Unstable terminal reading cannot auto-complete configured weighing. |
| IOT-T032 | Weight raw preservation | Selected gross/tare business values reference immutable raw readings. |
| IOT-T033 | Weight manual fallback | Manual reading works with reason when device adapter down. |
| IOT-T034 | LoRaWAN decoder version | Historical measurement retains decoder/profile version. |
| IOT-T035 | LoRaWAN key secrecy | Root/session key absent from normal API/log/support bundle. |
| IOT-T036 | OPC UA Bad status | Bad StatusCode maps to BAD quality and cannot auto-trigger protected fact. |
| IOT-T037 | OPC UA source/server time | Both time semantics retained where provided. |
| IOT-T038 | OPC UA insecure profile | Production config refuses anonymous/no-security default unless explicit legacy exception. |
| IOT-T039 | Command permission | User lacking DEVICE_COMMAND cannot issue command. |
| IOT-T040 | Command expiry | Expired offline command is not executed after reconnect. |
| IOT-T041 | Non-idempotent command | Automatic retry blocked/restricted for NON_IDEMPOTENT command. |
| IOT-T042 | Command correlation | Result maps to exact commandId. |
| IOT-T043 | Safety boundary | Platform cannot bypass local safety interlock/E-stop. |
| IOT-T044 | AI actuation | AI recommendation alone cannot send device command. |
| IOT-T045 | Credential rotation | New credential works; old revoked; same canonical device ID. |
| IOT-T046 | Config rollback | Bad edge mapping can return to previous version. |
| IOT-T047 | Firmware checksum | Tampered firmware artifact rejected. |
| IOT-T048 | Firmware staged rollout | Failure threshold pauses cohort rollout. |
| IOT-T049 | Health offline | Missing heartbeat/messages transitions health after profile threshold. |
| IOT-T050 | Gap detection | Expected 1-min sample gap creates quality/health issue. |
| IOT-T051 | Stuck sensor | Constant-value rule creates issue without deleting values. |
| IOT-T052 | Evidence mapping | STOP signal creates evidence/candidate, not automatic breakdown reason. |
| IOT-T053 | Fuel evidence | Level jump may create anomaly/refuel candidate but not direct stock ledger movement. |
| IOT-T054 | Production counter | Pulse count supports output evidence but does not close production order. |
| IOT-T055 | Raw retention | Expiring generic raw telemetry does not delete domain fact that references retained evidence metadata. |
| IOT-T056 | Cloud filtering | Policy can replicate aggregate/business evidence without all raw high-frequency samples. |
| IOT-T057 | Enterprise steady ingest | 5M/day equivalent meets NFR without durable loss. |
| IOT-T058 | Enterprise burst | 2,000 msg/s 5 min survives without host OOM/loss. |
| IOT-T059 | Critical alert latency | Local device event -> NotificationCase P95 <=10s under target load. |
| IOT-T060 | Protocol isolation | Crashed/slow Modbus adapter does not stall core API or MQTT adapter. |
| IOT-T061 | Site isolation | Device from site A cannot bind/command asset outside permitted scope. |
| IOT-T062 | Device replacement | Replacing hardware preserves business asset and historical device lineage. |
| IOT-T063 | Calibration expiry | Expired calibration propagates quality/alert according to profile. |
| IOT-T064 | Offline LocalPrimary | External Internet outage does not stop local IoT ingest. |
| IOT-T065 | Cloud unavailable | Local replication outbox/backlog accumulates and recovers idempotently. |
| IOT-T066 | No public DB | IoT VLAN cannot reach PostgreSQL/Redis directly. |
| IOT-T067 | Support bundle | No device secret/root key in diagnostics. |
| IOT-T068 | Schema compatibility | Unknown incompatible schema version is rejected/actionable, not guessed. |
| IOT-T069 | Mapping effective dates | New register mapping affects new observations only unless explicit reprocess. |
| IOT-T070 | Audit commands | Commission/revoke/config/command lifecycle creates security/audit evidence. |

# 26. Machine-readable gateway profile

gateway_profile:
  code: FACTORY_EDGE_01
  version: 1

  transports:
    mqtt:
      enabled: true
      qosTelemetry: 1
      tls: true
    httpBatch:
      enabled: true

  adapters:
    - type: MODBUS_RTU
      port: /dev/ttyUSB0
      mapProfile: press_line_v3
    - type: MODBUS_TCP
      endpoint: 192.168.50.20:502
      mapProfile: energy_meter_v2

  buffer:
    maxBytes: 1073741824
    priorities: true
    overflow: DROP_OLDEST_NONCRITICAL

  upstream:
    mode: LOCAL_PRIMARY
    server: https://agro.local

# 27. Порядок реализации Codex

| Stage | Component | Deliverable |
| --- | --- | --- |
| I25-01 | IoT domain schema | Device/Gateway/Channel/Signal/Measurement/Health/Command entities. |
| I25-02 | Canonical ingest | MeasurementEnvelope validation/idempotency/batch receipts. |
| I25-03 | Normalization | Decoder/scaling/calibration/unit/quality pipeline. |
| I25-04 | Edge SDK/agent contract | Durable buffer, batching, health, config. |
| I25-05 | MQTT adapter | Broker/topic ACL/QoS/session integration. |
| I25-06 | HTTP gateway adapter | mTLS/token batch ingest. |
| I25-07 | Modbus adapters | TCP/RTU mappings/polling/endian/errors. |
| I25-08 | Physical I/O mapping | 4-20mA/digital/pulse canonical profiles. |
| I25-09 | GPS/scale adapters | Position/weight evidence contracts. |
| I25-10 | LoRaWAN/OPC UA adapters | Optional provider-neutral adapters. |
| I25-11 | Evidence mapping | Signal -> domain candidate/event/notification rules. |
| I25-12 | Command plane | Permission/expiry/result/security; read-only default. |
| I25-13 | Health/config/firmware | Fleet operations. |
| I25-14 | IoT acceptance/load suite | 70 tests + NFR burst/soak/chaos. |

# 28. Проверенные внешние источники

| ID | Source | URL/file | Use |
| --- | --- | --- | --- |
| S01 | ГОСТ Р 70924-2023 | https://protect.gost.ru/gost/details/0db75a14-eea2-4c5e-a9d4-5b9ca1b30bd6 | Действующая российская типовая архитектура IoT; введён 01.01.2024. |
| S02 | ГОСТ Р 71777-2024 | https://protect.gost.ru/gost/details/688f305a-5d92-4e1f-81be-01142ecf5a09 | Действующие IoT термины/определения; введён 01.02.2025. |
| S03 | ГОСТ Р 71840-2024 | https://protect.gost.ru/gost/details/3d5314ea-b553-419e-bc65-b3b44ce7e9a8 | IoT data exchange platform, Part 1 general requirements. |
| S04 | ISO/IEC 30141:2024 | https://www.iso.org/standard/88800.html | Current international IoT reference architecture; edition 2. |
| S05 | MQTT Version 5.0 OASIS Standard | https://www.oasis-open.org/standard/mqtt-v5-0-os/ | Publish/subscribe IoT transport. |
| S06 | Modbus Specifications | https://www.modbus.org/modbus-specifications | Application Protocol V1.1b3; Serial Line Guide V1.02 for new serial implementations; Modbus Security profile. |
| S07 | LoRaWAN 1.0.4 Specification Package | https://lora-alliance.org/resource_hub/lorawan-104-specification-package/ | 1.0.4 stack/certification package; reference implementation list includes RU864. |
| S08 | LoRaWAN Specification v1.1 | https://lora-alliance.org/resource_hub/lorawan-specification-v1-1/ | LoRaWAN 1.1 network/security model. |
| S09 | OPC UA 1.05 online specifications | https://profiles.opcfoundation.org/document/ | Current 1.05 specification family. |
| S10 | OPC UA Part 8 Data Access | https://reference.opcfoundation.org/specs/OPC-10000-8 | Release 1.05.07 dated 2026-04-15. |
| S11 | Document №12 | document_12_offline_sync_specification.docx | Offline/local-first authority model. |
| S12 | Document №19 | document_19_security_personal_data_esign_audit.docx | Device/security/IoT trust boundary. |
| S13 | Document №20 | document_20_non_functional_requirements.docx | Telemetry volume/latency NFR. |
| S14 | Document №23 | document_23_data_governance_master_data_source_of_truth.docx | Raw/normalized/business fact governance. |

# 29. Решения, считающиеся зафиксированными

IoT is optional and cannot become a prerequisite for core platform operation.

Canonical IoT model is vendor/protocol neutral.

Raw measurement, normalized measurement, business evidence and business fact are distinct layers.

All production device data enters through Edge/Ingest contracts; no device writes database/domain tables directly.

MQTT 5.0 + HTTPS batch are preferred upstream transports; Modbus/serial/analog/digital protocols terminate at edge adapters.

LoRaWAN and OPC UA are optional adapters, not domain dependencies.

LOCAL_PRIMARY receives IoT data locally before cloud replication.

At-least-once delivery and duplicated transport messages are expected; application idempotency is mandatory.

V1 actuator command plane is disabled/read-only by default and cannot replace safety controls.

Device commands require capability allowlist, permission, expiry, result correlation and audit.

Legacy industrial protocols are treated as untrusted and protected by gateway/network segmentation.

Raw telemetry retention can be shorter than business evidence/fact retention.

Hardware replacement never changes business asset identity or historical provenance.

# 30. Acceptance Criteria Документа №25

Registry contains 250 stable IOT-* requirements.

Canonical IoT domain contains 25 typed entities/contracts.

RS-485/Modbus, 4-20mA, PNP/NPN, pulse, GPS, weighing, MQTT, LoRaWAN and OPC UA integration boundaries are explicit.

Codex can implement an edge adapter without inventing measurement/business semantics.

Offline store-and-forward, idempotency, timestamps, calibration, quality and device security are explicit.

Business modules remain usable when IoT is absent.

Command/actuation and safety boundaries prevent accidental autonomous control.

70 mandatory acceptance tests are defined.

Document can be transformed into /iot/device-schema.yaml, measurement-envelope.schema.json, mqtt-topics.yaml, protocol-mappings and IoT test fixtures in final Codex pack.

Следующий документ №26 - Production Integration / MES-lite / OEE / Retrofit. Там свяжем IoT Contract с реальным цехом: линии, станки, RUN/STOP/IDLE, микропростои, причины, счётчики, смены, выпуск/брак, сырьё, OEE, bottleneck, retrofit без вмешательства в PLC и интеграция с существующим оборудованием.